DRAFT AIS-062 (Rev.2)/ DF
December 2025
DRAFT AIS-062 (Rev.2)/ DF
December  2025

AUTOMOTIVE INDUSTRY STANDARD

Performance Requirements of Lighting and Light-Signalling Devices for Agricultural Tractors and Construction Equipment Vehicles (CEVs)
(Revision 2)
Date of hosting on website: 16th December, 2025

Last date for comments: 14th January, 2026
 INTRODUCTION

The Government of India felt the need for a permanent agency to expedite the publication of standards and development of test facilities in parallel when the work on the preparation of the standards is going on, as the development of improved safety critical parts can be undertaken only after the publication of the standard and commissioning of test facilities. To this end, the erstwhile Ministry of Surface Transport (MOST) vide order No. RT-11028/11/97-MVL dated                September 15, 1997. The standard prepared by AISC will be approved by the permanent CMVR Technical Standing Committee (CTSC). After approval, the Automotive Research Association of India, (ARAI), Pune, being the Secretariat of the AIS Committee, has published this standard. For better dissemination of this information ARAI has published this document on their Web site.

Performance of lighting and light-signalling devices for Agricultural Tractors is a safety requirement. AIS-062 covering these requirements has been published in 2004 and implemented in October 2005 vide notification GSR 206 (E) dated         1st April 2005.

With a continuous change in European Regulations to cater for new technologies and enhanced safety requirements, it was felt necessary to upgrade this standard to the level of corresponding ECE regulations.
The AISC panel and the Automotive Industry Standard Committee (AISC) responsible for preparation of this standard is given in Annexure 1 and Annexure 2 respectively.
Performance Requirements of Lighting and Light-Signalling Devices for Agricultural Tractors and Construction Equipment Vehicles (CEVs)
INDEX

	Paragraph No.
	Contents
	Page no.

	1
	Scope
	7/13

	2
	References 
	7/13

	3
	Definitions
	8/13

	4
	Requirements 
	8/13

	5
	Transitional Provisions
	9/13

	6
	Establishing  Compliance  of “E”/“E”  Approved

Lighting and Light Signalling Devices to this standards
	10/13

	7
	Amendments to ECE regulations after the level described in introduction
	10/13

	
	Annexures
	

	Annexure 1
	Composition  of  AISC panel  on Performance Requirements of  Lighting and Light-Signalling Devices for Agricultural Tractors and Construction Equipment Vehicles (CEVs)

	11/13

	Annexure  2
	Automotive Industry Standards Committee composition
	13/13


Performance Requirements of
 Lighting and Light-Signalling Devices for Agricultural Tractors and Construction Equipment Vehicles (CEVs)
1.0               Scope

               This standard lays down performance requirements of Lighting and     Light- signalling devices for Agricultural tractors and Construction Equipment Vehicles (CEVs).
2.0    Reference

The   following   standards   are   necessary   adjuncts   to   this     standard.

	AIS-030 (Rev.2):
20XX 
	Installation Requirements of Lighting and Light - Signalling Devices for Agricultural Tractors



	AIS-008(Rev.1):

2010
	Installation Requirements of Lighting and   Light-Signalling Devices for Motor Vehicle having more than Three Wheels, Trailer and Semi-Trailer excluding Agricultural Tractor and Special Purpose Vehicle 

	AIS-010 (Part 1) (Rev.1):2010 
	Provisions  concerning  the  Approval  of   Headlamps emitting an   Asymmetrical   Passing Beam or a Driving Beam or both and equipped with Filament Lamps and/or LED Modules 



	AIS-010(Part2) (Rev.1):2010 
	Provisions  concerning  the  Approval  of Headlamps emitting a Symmetrical  Passing Beam or a  Driving   Beam   or Both and equipped with Filament Lamps or Gas-Discharge Light Sources  



	AIS-010 (Part 5)

(Rev.1):2010 
	Requirements of  Chromaticity Co-ordinates of  Colour of  Light emitted from Lighting and Light-Signalling Devices 



	AIS-012(Part 1) (Rev.1): 2011 
	Performance Requirements  for Front Fog Lamps  for Motor Vehicles 



	AIS-012(Part 2) (Rev. 1):2011 
	Performance Requirements  for Rear Fog Lamps for Motor Vehicles 



	AIS-012 (Part 4) (Rev. 1):2011 
	Performance Requirements for Rear Registration Plate (mark) Illuminating Lamps  Rear Registration Plates for Motor Vehicles 



	AIS-012 (Part 5) (Rev. 1):2011 
	Performance Requirements  for Direction Indicators for  Motor Vehicles 



	AIS-012 (Part 6) (Rev. 1):2011 
	Performance Requirements for Front and Rear Position (Side) Lamps, Stop-Lamps and End-outline Marker Lamps for Motor Vehicles 



	AIS-012 (Part 7) (Rev. 1):2011 
	Performance Requirements for Reversing Lamps for Motor Vehicles 



	AIS-012 (Part 8) (Rev. 1):2011 
	Performance Requirements for Parking Lamps for Motor Vehicles 



	AIS-012 (Part 9) (Rev. 1):2011 
	Performance Requirements for Side-Marker Lamps for Motor Vehicles 



	AIS-057 ( Rev 1) : 2010
	Performance Requirements for Retro-Reflecting Devices for Power-Driven Vehicles and their Trailers

	AIS-088
	Performance Requirements of

Rear Marking Plates (Rear Warning Triangles) for Automotive Vehicles, Agricultural Tractors their Trailers and Semi-Trailers

	IS/ISO 12509 :2004
	Earth-moving machinery — Lighting, signalling, and marking lights, and reflex-reflector devices.

	AIS-012 (Part 10) (Rev 1) : 2011
	Performance Requirements for Daytime Running Lamps for Motor Vehicles

	AIS-012 (Part 3) (Rev.1) : 2011
	Performance Requirements for Cornering Lamps
for Motor Vehicles

	AIS 198
	Uniform Provisions Concerning the Approval of Light Signalling Devices and Systems for Power-Driven Vehicles

	AIS 199
	Uniform provisions concerning the approval of road illumination devices (lamps) and systems for power-driven vehicles

	AIS 200
	Uniform provisions concerning the approval of retro-reflective devices and markings for power-driven vehicles and their trailers


3.0    
   Definitions 

3.1
In addition to the following, definitions referred in AIS- 030 Rev 2 : 2025, AIS- 008 (Rev.3), IS/ISO 12509:2004, AIS- 012 (Part 1 to 10) (Rev. 1):2011, AIS 198, AIS 199 and AIS 200 shall be applicable.
   3.1.1    “Work Lamp (Plough lamp) of Different Type” means work lamp    Plough lamp) which differ in such essential respects as:
3.1.1.1.   The trade name or mark;

3.1.1.2
The material constituting the lenses and coating if any:

4.0          REQUIREMENTS 
   Lighting and light-signalling devices fitted on agricultural tractors and Construction Equipment Vehicles (CEVs) as specified in 1.0 shall meet the requirements of specified AIS given in Table 1 below:  
Table 1

Requirements for Performance of Lighting and 
Light-signalling devices for Agricultural Tractor

	Sr.

No.
	Lighting and Light-signalling Device
	AIS Reference

	1
	Main-beam headlamp and dipped-beam headlamp equipped with Filament Lamps and/or LED Modules

	AIS-010 (Part 1) (Rev.1) : 2010 

OR AIS 199


	2
	Main-beam headlamp and dipped-beam headlamp equipped with Filament Lamps or Gas-Discharge Light Sources

	AIS-010 (Part 2) (Rev.1) : 2010 

OR AIS 199


	3
	Front fog lamp (Only applicable for agricultural tractors, if fitted)

	AIS-012 (Part 1) (Rev.1) : 2011 OR AIS 199


	4
	Rear fog lamp, if fitted
	AIS-012 (Part 2)(Rev.1): 2011 OR AIS 198


	5
	Cornering lamps (Only applicable for agricultural tractors, if fitted)
	AIS-012 (Part 3) (Rev.1) : 2011 OR AIS 199

	6
	Rear registration plate (mark)

Illuminating lamp


	AIS-012 (Part 4) ( Rev 1) :2011 OR AIS 198

	7
	Front and rear direction indicator lamp and hazard warning lamp


	AIS-012 (Part 5)( Rev 1) : 2011 OR AIS 198



	8
	Front and rear position lamp, stop lamp, end-outline marker lamp
	AIS-012 (Part 6) ( Rev 1): 2011 OR AIS 198

	9
	Reversing lamp
	AIS-012 (Part 7) (Rev 1) : 2011 OR AIS 198



	10
	Parking lamp
	AIS-012(Part 8) (Rev 1) : 2011 OR AIS 198

	11
	Side-Marker lamps (Only applicable for agricultural tractors, if fitted)
	AIS-012 (Part 9) (Rev 1) : 2011 OR AIS 198

	12
	Daytime Running lamps (Only applicable for agricultural tractors, if fitted)
	AIS-012 (Part 10) (Rev 1) : 2011 OR AIS 198

	13
	Front, Rear and side  retro-reflector (non triangular) (Only applicable for agricultural tractors, if fitted)
	AIS-057 ( Rev 1) :2010 OR AIS 200

	14
	Rear warning triangle, if fitted
	AIS-088 : 2005 OR AIS 200

	15
	Conspicuity markings (if fitted)
	AIS 090 OR AIS 200

	16
	Exterior Courtesy Lamp (Only applicable for agricultural tractors, if fitted)
	---

	17
	Manoeuvring lamps (Only applicable for agricultural tractors, if fitted)
	---



	18
	Signalling panel and signalling foil (Only applicable for agricultural tractors, if fitted)
	---

	19
	Work Lamps (if fitted)
	---


5.0         TRANSITIONAL PROVISIONS
	5.1

5.2

5.2.1

5.2.2
	For new type of lamps, the transitional provision shall be as per AIS 000 as amended from time to time, unless otherwise stated.
For existing type of lamps, the transitional provision shall be as given below:

On and from 24 months from date of implementation of AIS-198, Test Agency shall not grant approvals to AIS 012 (Part 2) (Rev.1), AIS-012 (Part 4) (Rev. 1), AIS-012 (Part 5) (Rev. 1), AIS-012 (Part 6) (Rev. 1), AIS-012 (Part 7) (Rev. 1), AIS-012 (Part 8) (Rev. 1), AIS-012 (Part 9) (Rev. 1) andAIS-012 (Part 10) (Rev. 1) as applicable.

On and from 24 months from date of implementation of AIS-199, Test Agency shall not grant approvals to AIS-010 (Rev.1) (Part 1, 2, 4 and 5), AIS-012 (Rev.1) (Part 1), AIS-012 (Rev.1) (Part 3) and AIS-127, as applicable.

On and from 24 months from date of implementation of AIS-200, Test Agency shall not grant approvals to AIS-057 (Rev.1), AIS-022, AIS088 (Rev.1), AIS-089 (Rev.1) and AIS-090 (Rev.1), as applicable.

On and from date of implementation of AIS-198, Test Agency shall not refuse to grant extensions of approval to AIS 012 (Part 2) (Rev.1), AIS-012 (Part 4) (Rev. 1), AIS012 (Part 5) (Rev. 1), AIS-012 (Part 6) (Rev. 1), AIS-012 (Part 7) (Rev. 1), AIS-012 (Part 8) (Rev. 1), AIS-012 (Part 9) (Rev. 1) and AIS-012 (Part 10) (Rev. 1) as applicable.

On and from date of implementation of AIS-199, Test Agency shall not refuse to grant extensions of approval to AIS-010 (Rev.1) (Part 1, 2, 4 and 5), AIS-012 (Rev.1) (Part 1), AIS012 (Rev.1) (Part 3) and AIS-127, as applicable.

On and from date of implementation of AIS-200, Test Agency shall not refuse to grant extensions of approval to AIS-057 (Rev.1), AIS-022, AIS-088 (Rev.1), AIS-089 (Rev.1) and AIS-090 (Rev.1), as applicable.


	6.0

6.1
	ESTABLISHING  COMPLIANCE  OF “E”/“e”  APPROVED

LIGHTING AND LIGHT SIGNALLING DEVICES TO THIS STANDARDS

As an exception to 7.4 of AIS-037, (or related administrative decisions) for certifying compliance of “E”/”e” approved Lighting and Light Signaling devices to this standard, the test as per relevant clause of the relevant standard shall be carried out by testing agency


7.0 
AMENDMENTS TO ECE REGULATIONS AFTER THE LEVEL DESCRIBED IN INTRODUCTION
	7.1
	Acceptance of changes in ECE regulations after the level described in  para 0.3 of introduction shall be as per AIS-000, as amended from time to time, as applicable, unless otherwise stated in this standard.




ANNEXURE 1

 (See introduction)

COMPOSITION  OF  AISC PANEL  ON

LIGHTING  AND  LIGHT-SIGNALLING  DEVICES FOR 

AGRICULTURAL TRACTOR*

	Convener

	

	Mr. Phillip Koshi 
	Sonalika Tractors (TMA)

	         Members
	Representing

	Dr. B. V. Shamsundara
	ARAI

	Ms. Jyoti M. Kirve
	ARAI

	Mr. Padmakumar V. Parlikar
	ARAI

	Mr. Kamalesh B. Patil
	ARAI

	Head ED
	CIRT

	Mr. Manish Sharma
	ICAT

	Mr. Kiran Mulki
	NATRAX

	Mr. Baloo Chavhan
	Mahindra & Mahindra Ltd. (TMA)

	Mr. Pradeep Shinde
	Mahindra & Mahindra Ltd. (TMA)

	Mr. Sujit Hinge
	CNH Industrial India Pvt. Ltd. (TMA)

	Mr. Venkatesan V
	TAFE Ltd. (TMA)

	Mr. Ashok Kumar Singh
	Escort Kubota Ltd. (TMA)

	Mr. Ashish Kr. Mallarh
	Escort Kubota Ltd. (TMA)

	Mr. Anayat Beigh
	Swaraj Tractors (TMA)

	Mr. Mansingh Jagdale
	John Deere India Pvt. Ltd. (TMA)


 *   At the time of approval of this Automotive Industry Standard (AIS)

ANNEXURE 2
(See introduction)

COMMITTEE COMPOSITION *

Automotive Industry Standards Committee

	Chairperson
	

	Dr. Reji Mathai
	Director, The Automotive Research Association of India

	Members
	Representing

	Representative from
	Ministry of Road Transport and Highways                                       

	Representative from
	Ministry of Heavy Industries 

	Representative from
	Office of the Development Commissioner, MSME, Ministry of        Micro, Small and Medium Enterprises

	Shri Shrikant R. Marathe
	Former Chairman, AISC

	Head-TED
	Bureau of Indian Standards

	Director
	Central Institute of Road Transport

	Director
	Global Automotive Research Centre

	Director
	International Centre for Automotive Technology 

	Director
	Indian Institute of Petroleum

	Director
	National Automotive Test Tracks

	Director
	Vehicles Research and Development Establishment

	Director
	Indian Rubber Manufacturers Research Association 

	Representatives from
	Society of Indian Automobile Manufacturers 

	Representatives from
	Tractor and Mechanization Association 

	Representatives from
	Automotive Components Manufacturers Association of India

	Representative from
	Indian Construction Equipment Manufactures' Association 

	Member Secretary
	

	Shri Vikram Tandon
	The Automotive Research Association of India


*   At the time of approval of this Automotive Industry Standard (AIS)

Page 13 of 13

