| General | General | Comments /Suggestions |
| --- | --- | --- |
| # Section 100 Original 2013 VERSION # 101 INTRODUCTION 101.1 These specifications shall apply to all such road and bridge works as are required to be executed under the Contract or otherwise directed by the Engineer-in-Charge (hereinafter referred to as the Engineer). In every case, the work shall be carried out to the satisfaction of the Engineer and conform to the location, lines, dimensions, grades and cross-sections shown on the drawings or as decided by the Engineer. The quality of materials, processing of materials as may be needed at the site, salient features of the construction work and quality of finished work, measures for safety of workers and public. Traffic arrangements during execution shall comply with the requirements set forth in succeeding sections. Where the drawings and Specifications describe a portion of the work only in general terms, and not in complete detail, it shall be understood that only the sound engineering practice is to prevail, materials and workmanship of the best quality are to be employed and the instructions of the Engineer are to be fully complied with. 101.2 A list of Indian Roads Congress (IRC) Specifications and recommended Codes of Practice which have been referred in these Specifications is given at Appendix-1. The latest edition of all Specifications/Standards/Codes of IRC till 60 (sixty) days before the final date of submission of the tender, shall be adopted. 101.3 In case of any conflict or inconsistency in the provisions of the applicable Specifications/Standards/Codes of IRC, provisions contained in these Specifications shall apply. # 102 DEFINITIONS The words like Contract, Contractor, Engineer (synonymous with Engineer-in-charge), Drawings, Employer, Government, Works, and Work Site used in these Specifications. shall be considered to have the meaning as understood from the definitions of these terms are given in the General Conditions of Contract . The various elements in the cross-section of a road referred to in these specifications are shown in the cross-sections in Fig. 100.1, 100.2 and 100.3. are defined The following abbreviations shall have the meaning as set forth below: - AASHTO : American Association of State Highway and Transportation Officials ASTM: American Society for Testing and Materials BS : British Standard published by TheBritish StandardsInstitution BIS : Bureau of Indian Standards BOQ: Bill of Quantities CBR: California Bearing Ratio IRC: Indian Roads Congress IS: Indian Standard published by the Bureau of Indian Standards ISO: International Organization for Standardization QA: Quality Assurance Fig. 100.1 Terms Used in the Specifications to Describe Road Cross-Section Elements with a Two Lane Flexible Pavement Fig. 100.2 Terms Used in the Specifications to Describe Road Cross-Section Elements with a Two Lane Concrete Pavement (in open country in Isolated Built up Area) Fig. 100.3 Terms used in the Specifications to Describe Road Cross-Section Elements of a Dual Carriageway of Flexible Pavement Treated shoulders shown in the cross-section shall be of following types: - - “Unpaved shoulder shoulders which have select gravel/moorum, any other compacted granular layer or bricks. # ii) Paved” shoulders which have bituminous surfacing over granular layers. - Tied shoulders of same thickness as the of carriage way.: # 105 SCOPE OF WORK ## 105.1 The work to be carried out under the Contract shall consist of the various items as generally described in the Contract Documents as well as in the Bill of Quantities furnished in the Contract Documents. ## 105.2 Conformity with Drawings/Allowable Deviations 105.2.1 All works performed and all materials furnished shall be in conformity with the lines, grades, typical sections, dimensions, material requirements, and tolerances shown in the drawings or as indicated in the Specifications. ## 105.2.2 The works to be performed shall also include all general works preparatory to the construction of roads, bridges, highway, canal crossings, drainage and all other related works. The works shall include work of any kind necessary for the due and satisfactory construction, completion and maintenance of works to the intent and meaning of the drawings and these Specifications and further drawings and orders that may be issued by the Engineer from time to time. ## The scope of work shall include compliance by the Contractor with all Conditions of Contract, whether specifically mentioned or not in the various Sections of these Specifications, all materials, apparatus, plant, equipment, tools, fuel, water, strutting, timbering, transport, offices, stores, workshop, staff, labour and the provision of proper and sufficient protective works, diversions, temporary fencing and lighting. It shall include all works related to safety of road user. It shall also include safety of workers at construction site, , first-aid equipment, suitable , accommodation for the staff , and workmen with adequate sanitary arrangements, the effecting and maintenance of Insurances, the payment of all wages, salaries, fees, royalties, duties or other charges arising out of the erection of works and the regular clearance of rubbish, reinstatement and clearing-up of the site as may be required on completion of works, safety of the public and protection of the works and adjoining land/structures. ## 105.3 The Contractor shall ensure that all actions are taken to build in quality assurance (QA) in the planning, management and execution of works. The quality assurance shall cover all stages of work such as setting out, selection of materials, selection of construction methods, selection of equipment and plant, deployment of personnel and supervisory staff, quality control testing, etc. The QA programme shall cover the details as per IRC:SP:47 and IRC:SP:57. These shall broadly cover quality assurance aspects of all services rendered, all items to be supplied and all activities to be performed under the contract including temporary structures and equipment which will influence the quality of the completed works or the progress of the contract. ## As a minimum it shall cover the following: i) Organisation and management responsibility, ii) Document and data control, iii) Construction programme, v) Method statement, vi) Process control, vii) Working, inspection, testing and documentary procedures, viii) Arrangement for smooth and safe traffic flow during construction and maintenance, ix) Control and documentation of purchasing and handling of materials, x) Maintenance of records for non-conformity and timely corrective actions, xi) Internal quality audit, xii) Training of staff, xiii) Environment Management Plan (EMP). The QA plan shall be submitted to the Engineer for approval, not later than 28 days from the date of signing of the contract agreement. The work of building in quality assurance shall be deemed to be covered in the scope of the work. ## 105.4 The Contractor shall furnish, at least 7 days in advance, unless otherwise stipulated in the contract, his programme of commencement of each item of work, including the method statement including deployment of plant and equipment for the works included in the contract and any other work for which the Engineer may demand the method statement He shall provide all information to the satisfaction of the Engineer to ensure its adequacy. The sole responsibility for the safety and adequacy of the methods adopted by the Contractor will, however, rest on the Contractor, irrespective of any approval given by the Engineer. ## 105.5 Inspection of Materials before Incorporation ## 105.5.1 All materials shall be inspected, tested and accepted by the Engineer as per these specifications, before incorporation in the work. The frequencies and methods of sampling and testing materials, including those required for definite purpose and not covered by these specifications shall be in accordance to the relevant IRC or BIS or AASHTO/ASTM/BS Standards in order of priority. ## 105.5.2 All materials or work not conforming to the requirements of the Specifications shall be considered unacceptable and rejected. The unacceptable materials or work that are rejected shall be immediately removed, unless the defects are corrected and approved by the Engineer. If the Contractor fails to comply promptly with any order of the Engineer made under the provisions of this Clause, the Engineer has the authority to remove and replace unacceptable materials or work and to deduct from due to the Contractor. the cost of removal and replacement. ## 105.6 Inspection of Materials at Source ## The Engineer may choose to inspect material at source of supply. In the event, the following conditions shall be met. ## a) The Contractor and the manufacturer of material shall assist and co-operate with the Engineer in carrying out the inspection. ## b) The Engineer shall have right to enter areas of plant where the manufacture or production of material is carried out. ## 105.7 Delivery, Storage and Handling of Materials ## 105.7.1 All materials shall be handled and stored in appropriate manner to preserve their quality and fitness for the work. During the handling of all aggregates or other construction materials, special care shall be taken to prevent contamination. Furthermore, aggregate shall be handled in such a manner as to prevent segregation. ## 105.7.2 Vehicles used in transporting construction material shall be kept clean and in proper working condition so as to prevent the loss of materials during transportation and meet the requirements of the Specifications. ## 105.7.3 The Contractor may be allowed to store materials and equipment within the at location approved by the Engineer, but shall be responsible for the restoration and repair of any damage to plantation, signs, property or any assets resulting from such operations. Any additional space that may be needed for storage purposes and for placing of plant and equipment shall be provided by the Contractor at no additional cost to the Employer. ## 105.8 Materials Furnished by the Employer ## When the Contract provides that certain materials required to complete the work will be supplied by the Employer, such material will be delivered or made available to the Contractor at the location(s) specified in the Contract. ## The Contractor shall be responsible for all damages occurring to the materials furnished by the Employer while the materials are in his possession. Any demurrage or storage charges shall also be the responsibility of the Contractor. ## The Contractor shall include the cost of handling, transportation and placing of all Employer furnished materials in the Contract unit price for the relevant pay item. ## 105.9 Laws to be Observed ## The Contractor shall observe and comply with all Central and State laws, local laws and ordinance which affect those employed on the work or affect the conduct of the work. The Contractor shall provide all safeguards, safety devices, and protective equipment and take any other actions necessary for safety and health of employees on the project. ## 105.10 Patented Devices, Materials and Processes ## If the Contractor is required or desires with the approval of the Engineer to use any design, device, material or process covered by trademark, patent or copyright, the Contractor shall obtain the right for its use by legal agreement with the patentee or owner. A copy of the agreement shall be furnished to the Engineer. Contract prices shall include all royalties and costs arising from patents, trademarks and copyrights. # 103 MATERIALS AND TEST STANDARDS The relevant standards for materials, as well as the testing procedures, have been indicated at appropriate places in the specifications. A list of these standards with their full title are included at Appendix 2. # 104 SIEVE DESIGNATIONS The sieve designations referred to in the Specifications correspond to those specified by Bureau of Indian Standards in IS:460. Table 100-1 gives the list of the commonly used IS sieves. Table 100-1 DESIGNATION OF TEST SIEVES IS Designation conforming to IS:460 # 106. CONSTRUCTION EQUIPMENT In addition to the conditions indicated in the Contract Documents, the following conditions regarding use of equipment in works shall be satisfied: ### a) The Contractor shall be required to give a trial run of the equipment for establishing their capability to achieve the laid down Specifications and tolerances to the satisfaction of the Engineer before commencement of the work; ### b) All equipment provided shall be of proven efficiency and shall be operated and maintained at all times in a manner acceptable to the Engineer; ### c) Plants, equipment and instruments provided shall have adequate sensitivity, facility for calibration to desired level and shall be robust; ### d) Plant, equipment and instrument provided shall have data logging arrangement and control systems to enable automatic feedback control of process; ### e) Plants, equipment and instruments provided shall have adequate safety features and pollution control devices; ### f) Plant, equipment and instruments provided shall be operated by skilled and qualified operators; ### g) All the plant/equipment to be deployed on the works shall be got approved from the Engineer for ensuring their fitness and efficiency before commencement of work; ### h) Any material or equipment not meeting the approval of the Engineer shall be removed from the site forthwith; ### i) No equipment shall be removed from site without permission of the Engineer; ### j) The Contractor shall also make available stand by equipment and spare parts; and ### k) The Contractor shall also make available equipment for site quality control work as directed by the Engineer. # 107 DRAWINGS ## 107.1 The drawings provided in the Tender Documents shall be used as reference only. The Contractorshall study the nature and type of work and ensure that the rates and prices quoted by him in the Bill of Quantities have due consideration of the site and complexities of work involved during actual execution/construction. ## 107.2 The Contractor based on his surveys and investigations, shall submit the working drawings (hard and soft copy) to the Engineer for each activity at least 45 days in advance of the scheduled date to the start of the activity as per his approved work programme. The working drawings shall clearly show the modifications, if any, proposed with reference to corresponding tender drawings. The Engineer shall review the working drawings including the modifications proposed, if any, revise the drawings, if required, approve and issue to the Contractor two copies of Good for Construction (GFC) drawings at least 28 15days in advance of the scheduled date of the start of the activity. ## 107.3 Examination and/or approval by the Engineer of any drawings or other documents submitted by the Contractor shall not relieve the Contractor of his responsibilities or liabilities under the Contract. ## 107.4 The tendered rates/prices for the work shall be deemed to include the cost of preparation, supply and delivery of all necessary drawings, prints, tracings and negatives which the Contractor is required to provide in accordance with the Contract . # 108 SITE INFORMATION ## 108.1 The information about the site of work and site conditions in the Tender Documents is given in good faith for guidance only but it shall be the responsibility of the Contractor to satisfy himself regarding all aspects of site conditions. ## 108.2 The location of the works and the general site particulars are as shown in the Site plan/Index plan enclosed with the Tender Documents. ## 108.3 Whereas the to the bridge sites/road works shall be provided to the Contractor by the Employer, the Contractor shall have to make his own arrangement for the land required by him for site offices, field laboratory, site for plants and equipment, maintenance and repair workshop, construction workers’ camp, , stores, etc. # 109. SETTING OUT ## 109.1 The Contractor shall establish working bench marks tied with the Reference bench marks in the area soon after taking possession of the site. The Reference bench marks for the area shall be as indicated in the Contract Documents and the values of the same shall be obtained by the Contractor from the Engineer. The working bench marks shall be at the rate of four per km and also at or near all drainage structures, over-bridges and underpasses. The working bench marks /levels should be got approved from the Engineer. Checks must be made on these bench marks once every month and adjustments, if any, got approved from the Engineer and recorded. An up-to-date record of all bench marks including approved adjustments, if any, shall be maintained by the Contractor and also a copy supplied to the Engineer for his record. ## 109.2 The lines and levels of formation, side slopes, drainage works, carriageways and shoulders shall be carefully set out and frequently checked, care being taken to ensure that correct gradients and cross-sections are obtained everywhere. ## 109.3 In order to facilitate the setting out of the works, the centre line of the carriageway or highway must be accurately established by the Contractor and approved by the Engineer. It must then be accurately referenced in a manner satisfactory to the Engineer, at every 50 m intervals in plain and rolling terrains and 20 m intervals in hilly terrain and in all curve points as directed by the Engineer, with marker pegs and chainage boards set in or near the fence line, and a schedule of reference dimensions shall be prepared and supplied by the Contractor to the Engineer. These markers shall be maintained until the works reach finished formation level and are accepted by the Engineer. ## 109.4 On construction reaching the formation level stage, the centre line shall again be set out by the Contractor and when approved by the Engineer, shall be accurately referenced in a manner satisfactory to the Engineer by marker pegs set at the outer limits of the formation. ## 109.5 No reference peg or marker shall be moved or withdrawn without the approval of the Engineer and no earthwork or structural work shall commence until the centre line has been referenced. ## 109.6 The Contractor will be the sole responsible party for safe-guarding all survey monuments, bench marks, beacons, etc. The Engineer will provide the Contractor with the data necessary for setting out the centre line. All dimensions and levels shown on the drawings or mentioned in documents forming part of or issued under the Contract shall be verified by the Contractor on the site and he shall immediately inform the Engineer of any apparent errors or discrepancies in such dimensions and levels. The Contractor shall, in connection with the staking out of the centre line, survey the terrain along the road and shall submit to the Engineer for his approval, a profile along the road centre line and cross-sections at intervals as required by the Engineer. ## The construction staking shall be done by personnel who are trained and experienced in construction layout and staking of the type and kind required in the Contract. ## Field notes shall be kept in standard, bound field notebooks asapproved by the Engineer. Field notes shall be subject to inspection by the Engineer and shall be the property of the Employer. The Contractor shall correct any deficient staking or construction work which resulted from inaccuracies in the staking operations or from the Contractor’s failure to report inaccuracies in the plans or survey data furnished by the Department. ## 109.7 After obtaining approval of the Engineer, work on earthwork can commence. The profile and cross-sections as per Section 305, shall form the basis for measurements and payment. The Contractor shall be responsible for ensuring that all the basic traverse points are in place at the commencement of the contract and, if any, are missing, or appear to have been disturbed, the Contractor shall make arrangements to re-establish these points. A “survey File” containing the necessary data will be made available for this purpose. If in the opinion of the Engineer, design modifications of the centre line or grade are advisable, the Engineer will issue detailed instructions to the Contractor and the Contractor shall perform the modifications in the field, as required, and modify the ground levels on the cross-sections accordingly as many times as required. ## There will be no separate payment for any survey work performed by the Contractor. The cost of these services shall be considered as being included in the rate of the items of work in the Bill of Quantities ## 109.8 Precision automatic levels, having a standard deviation of +2 mm per km, and fitted with micrometer attachment shall be used for all double run levelling work. Setting out of the road alignment and measurement of angles shall be done by using Total Station with traversing target, having an accuracy of one second. Measurement of distances shall be done preferably using precision instruments like Distomat. ## 109.9 The work of setting out shall be deemed to be a part of general works preparatory to the execution of work and no separate payment shall be made for the same. # 110 PUBLIC UTILITIES ## 110.1 Drawings scheduling the affected services like water pipes, sewers, oil pipelines, cables, gas ducts etc. owned by various authorities including Public Undertakings and Local Authorities included in the Contract Documents shall be verified by the Contractor for the accuracy of the information prior to the commencement of any work. ## . The Contractor shall notify all utility agencies who may have installation in the work area and secure their assistance in locating and identifying all utilities before starting any work that may cause any damage to such utilities. ## The Contractor / Engineer shall notify all utility agencies who may have installation in the work area and secure their assistance in locating and identifying all utilities before starting any work that may cause any damage to such utilities. ## The Contractor shall schedule work in such a manner as to protect existing utility facilities until they are relocated, abandoned or replaced. ## The Contractor shallensure that all utilities encountered within the i.e. OFC Cable, telephone, power, water supply, sewerage or any others, remain operational at all times. Any utility, if damaged, due to construction operation, shall be promptly repaired by the Contractor at his cost. ## 110.2 Notwithstanding the fact that the information on affected services may not be exhaustive, the final position of these services within the works shall be supposed to have been indicated based on the information furnished by different bodies and to the extent the bodies are familiar with the final proposals. The intermediate stages of the works are, however, unknown at the design stage, these being dictated by the Contractor’s methods of working. Accordingly, the Contractor’s programme must take into account the period of notice and duration of diversionary works of each body as given on the Drawings and the Contractor must also allow for any effect of these services and alterations upon the Works and for arranging regular meetings with the various bodies at the commencement of the Contract and throughout the period of the Works, the Contractor shall have no objection if the public utility bodies vary their decisions in the execution of their proposals in terms of programme and construction, provided that, in the opinion of the Engineer, the Contractor has received reasonable notice thereof before the relevant alterations are put in hand. ## 110.3 No removal of or alterations to the utility shall be carried out unless written instructions are issued by the Engineer. ## 110.4 Any services affected by the Works must be temporarily supported by the Contractor who must also take all measures reasonably required by the various bodies to protect their services and property during the progress of the Works. ## 110.5 The Contractor may be required to carry out certain works for and on behalf of various bodies, which he shall provide, with the prior approval of the Engineer. ## 110.6 The work of temporarily supporting and protecting the public utility services during execution of the Works shall be deemed to be part of the Contract and no extra payment shall be made for the same. ## 110.7 The Contractor shall be responsible to co-ordinate with the service providers for cutting of trees, shifting of utilities, removal of encroachments etc. to make site unencumbered for completion of work. This will include frequent follow-up meetings. Co-ordination for making project site unencumbered shall be deemed to be part of the Contract and no extra payment shall be made for the same. ## 110.8 In some cases, theContractor may be required to carry out the removal or shifting of certain services/utilities on specific orders from the Engineer for which payment shall be made to him. Such works, however, shall be taken up by the Contractor only after obtaining clearance from the Engineer and ensuring adequate safety measures. # 111 PRECAUTIONS FOR SAFEGUARDING THE ENVIRONMENT # 111.1 General ## The Contractor shall take all precautions for safeguarding the environment during the course of the construction of the works. He shall abide by all laws, rules and regulations in force governing pollution and environmental protection that are applicable in the area where the works are situated. # 111.2 Borrow Pits for Embankment Construction ## 111.2.1 Borrow pits shall be selected only after testing the suitability of materials for use in construction and shallnot normally be dug in the of the road. The stipulations in Section 305.2.2 shall govern. The borrow pits shall not be left in a condition likely to cause hazard to human and animal life. The Contractor shall seek prior approval from the concerned authorities for operating the borrow pits. # 111.3 Quarry Operations ## The Contractor shall obtain materials from quarries only after obtaining the consent of the Mining Department or other concerned authorities. The quarry operations shall be undertaken within the purview of the rules and regulations in force. # 111.4 Control of Soil Erosion, Sedimentation and Water Pollution ## The Contractor shall carry out the works in such a manner that soil erosion is fully controlled, and sedimentation and pollution of natural water courses, ponds, tanks and reservoirs is avoided. The stipulations in Clause 306 shall govern. Treatment of embankment and Roadside slopes for erosion control shall be as per IRC:56. # 111.5 Pollution from Plants and Batching Plants ## Stone crushing and screening plants, Bituminous hot-mix plants, concrete batching plants etc. shall be located sufficiently away from habitation, agricultural operations fields or industrial establishments. The locations shall be as permissible under the laws governed by local bodies/ administration of the area. The Contractor shall take every precaution to reduce the levels of noise, vibration, dust and emissions from his plants and shall be fully responsible for any claims or damages caused to the owners of property, fields and residences in the vicinity and violation of pollution control norms, if any. # 111.6 Substances Hazardous to Health ## The Contractor shall not use or generate any materials in the works which are hazardous to the health of persons, animals or vegetation. Where it is necessary to use some substances which can cause injury to the health of workers, the Contractor shall provide protective clothing or appliances to his workers. ## 111.7 Use of Nuclear Gauges ## Nuclear gauges shall be used only where permitted by the Engineer The Contractor shall provide the Engineer with a copy of the regulations governing the safe use of nuclear gauges he intends to employ and shall abide by such regulations. ## 111.8 Environmental Protection ## 111.8.1. The Contractor must take all reasonable steps to minimize dust nuisance during the construction of the works along the haul roads and the worksites by sprinkling water at a frequency specified by the Engineer. ## All existing highways and roads used by vehicles or equipments of the Contractor or any of his sub-contractors or suppliers of materials or plant, and similarly any new roads which are part of the works and which are being used by traffic, shall be kept clean and clear of all dust/mud or other extraneous materials dropped by the said vehicles. Similarly, all dust/mud or other extraneous materials from the works spreading on these highways shall be immediately cleared by the Contractor ## Clearance shall be effected immediately by sweeping and removal of debris, and all dust, mud and other debris shall be removed entirely from the road surface. Additionally, if so directed by the Engineer, the road surface shall be hosed or watered using suitable equipment. ## Damages to existing roads: Any structural or functional damages and loss of riding surface caused to the existing roads by the Contractor’s construction vehicles/ equipment shall be made good without any extra cost.. ## Compliance with the foregoing will not relieve the Contractor of any responsibility for complying with the requirements of any authority in respect of the roads used by him. # 111.8.2 Air Quality ## The Contractor shall device, and implements methods of working to minimize dust, gaseous and other air-borne emissions and carry out the Works in such a manner as to minimize adverse impacts on the air quality. ## The Contractor shall utilize effective water sprays during delivery, manufacture, processing and handling of materials when dust is likely to be created, and to dampen stored materials during dry and windy weather. Stockpiles of friable materials shall be covered with clean tarpaulins, with applications of sprayed water during dry and windy weather. Stockpiles of materials or debris shall be dampened prior to their movement, except where this is contrary to the Specification. ## Any vehicle with open load-carrying area used for transporting potentially dust-producing material shall have properly fitting side and tail boards. Materials having the potential to produce dust shall not be loaded to a level higher than the side and tail boards and shall be covered with clean tarpaulins in good condition. The tarpaulin shall be properly secured and extend at least 300 mm over the edges of the side and tail boards. # 111.8.3 Water Sources and Water Quality ## The Contractor shall provide independent sources of water supply, such as bore wells, for use in the Works and for associated storage, workshop and work force compounds. Prior approval shall be obtained from the relevant State Authorities and all installations shall be in compliance with local regulations. ## The Contractor shall protect all watercourses, waterways, ditches, canals, drains, lakes, reservoirs and the like from pollution as a result of the execution of the Works. All water and other liquid waste products like petroleum products and chemicalsarising on the Site shall be collected and disposed of at a location on or off the Site and in a manner that shall not cause either nuisance or pollution. ## The Contractor shall at all times ensure that all existing stream courses and drains within and adjacent to the Site are kept safe and free from any debris and any materials arising from the Works. The Contractor shall not discharge or deposit any matter arising from the execution of the Works into any water course except with the permission of the Engineer and the regulatory authority concerned. # 111.8.4 Construction Camps ## The construction camps shall conform to the State and National building regulations as applicable. The area for the storage of polluted materials shall be stored on impervious floors and shall be surrounded by impervious ditches in order to avoid spilling of polluted material to surrounding areas. Construction camps shall be properly arranged to avoid noise pollution to the nearby habitants and to avoid contamination of water courses from wastewater drainage. To prevent such contamination, waste water generated at the campsites shall be discharged into soak pits. Human excreta shall be treated though septic tanksprior to discharge and shall conform to directives and guidelines of the State, Water accumulated in tyres, empty vessels and containers of all nature will be regularly cleaned to avoid the related health hazards. The Contractor shall provide and maintain in a neat and sanitary condition accommodations for the use of the employees and workers as may be necessary to comply with the requirements of Central, State, and local regulations. # Spilling of oil and bituminous products during construction and transport shall be avoided to reduce the chances of contamination of surface as well as ground water. # 111.9 Occupational Health & Safety of the workforce ## The Contractor shall prepare and submit to the Engineer the Occupational Health & Safety Procedures / Practices for the workforce in all quarry sites, plant sites, work sites, camp sites, etc., in accordance with the applicable laws. # 111.10 Control and Disposal of Wastes ## The Contractor shall control the disposal of all forms of waste generated by the construction operations and in all associated activities. No uncontrolled deposition or dumping shall be permitted. Wastes to be so controlled shall include, but shall not be limited to, all forms of fuels and engine oils, all types of bitumen, cement, surplus aggregates, gravels, bituminous mixtures etc. The Contractor shall make specific provision for the proper disposal of these and any other waste products, conforming to local regulations and acceptable to the Engineer. # 111.11 Transport of Hazardous Materials ## Transport of hazardous materials, in bulk or in sealed containers, shall meet the requirements of the State regulations. Prior to ordering transport of hazardous material in bulk, the Contractor must obtain the approval of the relevant authority as well as of the Engineer. The transport of diesel, petrol, gaseous material, chemical and explosives for quarrying shall be governed by safety laws of the local authorities. Precautionary measures and conformity with regulations shall be stated in a Method Statement for the approval of the Engineer. Sealed containers of hazardous materials shall be stored in a well-ventilated room, well guarded and secured. # 111.12 Emergency Response ## The Contractor shall plan and provide remedial measures to be implemented in the event of occurrence of emergencies such as spillages of oil or bitumen or chemicals, fire. The Contractor shall provide the Engineer with a statement of the measures he intends to implement in the event of such an emergency, which shall include a statement of how he intends to provide personnel adequately trained to implement such measures. # 111.13 Measurement for Payment ## The compliance of all provisions made in this Clause 111 shall be deemed to be incidental to the work and no separate measurement or payment shall be made. The Contractor shall be deemed to have made allowance for all such compliance with these provisions in the preparation of his bid for items of work included in the Bill of Quantities and full compensation for such compliance shall be deemed to be covered by the bid price. # 112 ARRANGEMENT FOR TRAFFIC DURING CONSTRUCTION # 112.1 General The Contractor shall at all times, carry out work on the highway in a manner creating least interference to the flow of traffic while consistent with the satisfactory execution of the same. For all works involving improvements to the existing highway, , the Contractor shall, in accordance with the directives of the Engineer, provide and maintain, during execution of the work, a passage for traffic either along a part of the existing carriageway under improvement or along a temporary diversion constructed close to the highway. Before taking up any construction or maintenance operation, the Contractor shall prepare Traffic Management Plan for each work zone and submit it to the Engineer for prior approval. This plan should include inter alia : ### i) Provision of a qualified safety officer with support staff to serve as a site safety team ### ii) Provision of traffic safety devices and road signs in construction zones as per IRC:SP 55 and other relevant IRC Codes and para 112.4: ### iii) Safety measures for the workers engaged including personal protection equipment ### iv) First aid and emergency response arrangements ### v) Details and drawings of arrangements in compliance with other sub Sections of this Section. # 112.2. Passage of Traffic along a Part of the Existing Carriageway under improvement For widening/strengthening existing carriageway where part width of the existing carriageway is proposed to be used for passage of traffic, treated shoulders shall be provided on the side on which work is not in progress. The treatment to the shoulder shall consist of providing at least 150 mm thick granular (Wet Mix Macadam/ Water Bound Macadam) base course covered with bituminous premix carpet with seal coat / surface dressing, in a width of at least 1.5 m and the treated shoulder shall be maintained throughout the period during which traffic uses the same to the satisfaction of the Engineer. The continuous length, in which such work shall be carried out, would be limited normally to 500 m at a place. However, where work is allowed by the Engineer in longer stretches passing places at least 20 m long with additional paved width of 2.5 m shall be provided at every 0.5 km interval. In case of eccentric widening of existing two-lane to four-lane, the existing two-lane road shall be strengthened to carry the diverted traffic plus construction traffic plus normal traffic on the carriageway. Thereafter the work on construction of additional two-lane shall be taken up. In case of concentric widening, stipulations as in paragraph above shall apply. After the works are completed, with the approval of the Engineer, the treated shoulder shall be dismantled, the debris disposed of and the area cleared as per the direction of the Engineer. # 112.3 Passage of Traffic along a Temporary Diversion In stretches where it is not possible to pass the traffic on part width of the carriageway, a temporary diversion shall be constructed with 7 m carriageway and 2.5 m earthen shoulders on each side (total width of roadway 12 m) with the following provision for road crust in the 7 m width: ### i) Earthwork ### ii) 200 mm (compacted) granular sub-base ### iii) 225 mm (compacted) granular base course ### iv) Priming and Tack Coat and ### v) Premix carpet with Seal Coat/Mix Seal Surfacing/ The location of such stretch, alignment and longitudinal section of diversion including junctions and temporary cross drainage provision shall be as approved by the Engineer. # 112.4 Traffic Safety and Control The Contractor shall take all necessary measures for the safety of traffic during construction and provide, erect and maintain such barricades, including signs, marking, flags, lights and flagmen as per the work zone traffic management plan WTMP submitted by the Contractor and approved by the Engineer, referred to in Sub-Section 112.1. Before taking up any construction, an agreed phased programme for the diversion of traffic on the highway shall be as per the traffic management plan The Contractor shall conduct all operations to minimize any drop-offs (abrupt changes in roadway) exposed to traffic. Drop-offs in the travelled way shall be protected by a wedge of compacted stable material capable of carrying traffic (the wedge being 1 vertical to 4 horizontal or flatter). The Engineer shall authorize other methods, to protect drop-offs when conditions do not allow a wedge of compacted, stable material. Warning signs, barricades, warning lights, and all other traffic control devices shall not be removed if the hazard has not been eliminated. Only upon receipt of specific written authorization from the Engineer, the Contractor may remove or cease to maintain warning signs, barricades, warning lights, and all other traffic control devices. The barricades erected on either side of the carriageway/portion of the carriageway closed to traffic, shall be of strong design to resist violation, and painted with alternate black and white stripes. Red lanterns or warning lights of similar type shall be mounted on the barricades at night and kept lit throughout from sunset to sunrise. At the points where traffic is to deviate from its normal path (whether on temporary diversion or part width of the carriageway) the channel for traffic shall be clearly marked with the aid of pavement markings, painted drums or a similar device to the directions of the Engineer. At night, the passage shall be delineated with lanterns or other suitable light source including solar energy bulbs. One-way traffic operation shall be established whenever the traffic is to be passed over part of the carriageway inadequate for two-lane traffic. This shall be done with the help of temporary traffic signals or flagmen kept positioned on opposite sides during all hours. For regulation of traffic, the flagmen shall be equipped with red and green flags and lanterns/lights. On both sides, suitable regulatory/warning signs as approved by the Engineer shall be installed for the guidance of road users. On each approach, at least two signs shall be put up, one close to the point where transition of carriageway begins and the other 120 m away. The signs shall be of approved design and of reflective type, as directed by the Engineer. # 112.5 Maintenance of Diversions and Traffic Control Devices Signs, lights, barriers and other traffic control devices, adequate lighting and other arrangements, as well as the riding surface of diversions and treated shoulders shall be maintained in a satisfactory condition till such time they are required and as directed by the Engineer. The temporary travelled way shall be kept free of dust by frequent applications of water, if necessary. # 112.6 Measurements for Payment and Rate All arrangements, as contained in this section (112) for safety of road users, during construction including provision of temporary diversions/temporary cross drainage structures/treated shoulders shall be measured and paid as per the BOQ. However their maintenance, dismantling and clearing debris shall be considered as incidental to the Works and shall not be paid separately. # 113 GENERAL RULES FOR THE MEASUREMENT OF WORKS FOR PAYMENT # 113.1 General All measurements shall be made in the metric system. Different items of work shall be measured in accordance with the procedures set forth in the relevant Sections read in conjunction with the General Conditions of Contract. The same shall not, however, apply in the case of lumpsum contracts. All measurements and computations, unless otherwise indicated, shall be carried nearest to the following limits: - length and width 10 mm ### ii)height, depth or thickness of: #### a) earthwork, subgrade,5 mm #### b) sub-bases, bases, surfacing, 5 mm #### iii)structural members 2.5 mm ### iv) iv) areas 0.01 sqm ### v) v) volume 0.01 cum ## In recording dimensions of work, the sequence of length, width and height or depth or thickness shall be followed. # 113.2 Measurement of Lead for Materials Where lead is specified in the Contract for construction materials, the same shall be measured as described hereunder: Lead shall be measured over the shortest practicable route and not the one actually taken and the decision of the Engineer in this regard shall be taken as final. Distances upto and including 100 m shall be measured in units of 50 m, exceeding 100 m but not exceeding 1 km in units of 100 m and exceeding 1 km in units of 500 m, the half and greater than half of the unit shall be reckoned as one and less than half of the unit ignored. In this regard, the source of the material shall be divided into suitable blocks and for each block, the distance from the centre of placing pertaining to that block shall be taken as the lead distance. # 113.3 Measurement of Pavement Thickness for Payment on Volume Basis The finished thickness of sub-bases, base and bituminous layers and concrete courses to be paid on volume basis shall be computed in the following manner: Levels shall be taken before and after construction, at the grid of points 10 m centre-to-centre longitudinally in straight reaches and 5 m centre-to-centre at curves. Normally, on two-lane roads, the levels shall be taken at four positions transversely, at 0.75 m and 2.75 m from either edge of the carriageway and on single-lane roads, these shall be taken at two positions transversely, being at 1.25 m from either edge of the carriageway. For multi-lane roads, levels shall be taken at two positions transversely for each lane. The transverse position for levels shall be 0.75 m from either edge of the carriageway and the remaining locations shall be at equi-distance in the balance portion of carriageway. For paved shoulder an additional level shall be taken at the centre of the shoulder. Suitable references for the transverse grid lines should be left in the form of embedded bricks on both ends or by other means so that it is possible to locate the grid points for level measurements after each successive course is laid. For pavement courses laid only over widening portions, at least one line of levels shall be taken on each strip of widening, or more depending on the width of widening as decided by the Engineer. Notwithstanding the above, the measurements may be taken at closer intervals also, if so desired by the Engineer, the need for which may arise particularly in the case of estimation of the volume of the material for profile corrective course (leveling course). The average thickness of the pavement course in any area shall be the arithmetic mean of the difference of levels before and after construction at all the grid points falling in that area, provided that the thickness of finished work shall be limited to those shown on the drawings or approved by the engineer in writing. As supplement to level measurements, the Engineer shall have the option to take cores/make holes to check the depth of construction. The holes made and the portions cut for taking cores shall be made good by the Contractor by laying fresh mix/material including compacting as required at his-own cost immediately after the measurements are recorded. # 113.4 Checking of Pavement Thickness for Payment on Area / Volume Basis Where payment for any bituminous course in Section 500 is allowed to be made on the area basis, the Engineer may have its thickness checked with the help of a suitable penetration gauge at regular intervals or other means as engineer may direct. # 113.5 Measurement of Bituminous Courses for Payment on Weight Basis Plant-mixed bituminous materials for pavement courses, where specifically designated in the contract to be paid on weight basis, shall be weighed on accurate scales approved by the Engineer. Approved scales shall mean scales that are of size, capacity, kind and type suitable for the weighing to be done, and these shall be properly installed and maintained. Prior to the use of the scales and as frequently thereafter as the Engineer may deem necessary to ensure accuracy, the scales shall be checked and approved by the Engineer, or the Engineer may direct the Contractor to have the scales checked by other competent agency at the cost of the Contractor. Location of the scales shall be as designated by the Engineer. Trucks used for hauling the material to be weighed shall be weighed empty daily at such times as the Engineer directs, and each truck shall bear a plainly legible identification mark. For materials specified to be measured by weight, the Engineer will have the option to make measurements of the finished work by volume in accordance with Section 113.3 and such volumes shall be converted into weight for payment purposes. The factor for conversion from volume measurement to weight measurement shall be computed from the representative density of the compacted material at site determined at locations approved by the Engineer. # 114 SCOPE OF RATES FOR DIFFERENT ITEMS OF WORK ## 114.1 For item rate contracts, the contract unit rates for different items of work shall be payment in full for completing the work to the requirements of the Specifications including full compensation for all the operations detailed in the relevant Sections of these Specifications under “Rates”. In the absence of any directions to the contrary, the rates are to be considered as the full inclusive rate for completed work covering all labour, materials, wastage, temporary work, plant, equipment, over-head charges and profit as well as the general liabilities, performance of other obligations, insurance and risks arising out of the Conditions of Contract.. ## 114.2 The item rates quoted by the Contractor shall, unless otherwise specified, also include compliance with/supply of the following: ### i) General works such as setting out, clearance of site before setting out and clearance of works after completion; ### ii) A detailed programme using modern project management software for the construction and completion of the work giving, in addition to construction activities, detailed network activities for the submission and approval of materials, procurement of critical materials and equipment, fabrication of special products/equipment and their installation and testing, for all activities of the Engineer/Employer that are likely to affect the progress of work, etc., including updating of all such activities on the basis of the decisions taken at the periodic site review meetings or as directed by the Engineer; - Samples of various materials proposed to be used on the Works for conducting tests thereon as required as per the provisions of the contract - Design of mixes as per the relevant Sections of the Specifications giving proportions of ingredients, sources of aggregates and binder along with accompanying trial mixes as per the relevant Sections of these Specifications to the be submitted to the Engineer for his approval before use on the Works; ### v) Cost of laying trial stretches; ### vi) Detailed drawings as per Clause 107; - Detailed design calculations and drawings for all Temporary Works (such as form-work, staging, centering, specialized constructional handling and launching equipment and the like); - Detailed drawings for templates, support and end anchorage, details for pre-stressing cable profiles, bar bending and cutting schedules for reinforcement, material lists for fabrication of structural steel, etc; - Mill test reports for all mild and high tensile steel and cast steel as per the relevant provisions of the Specifications; - Testing of various finished items and materials including bitumen, cement, concrete, bearings as required under these Specifications and furnishing test reports/certificates. - Inspection Reports in respect of formwork, staging, reinforcement and other items of work as per the relevant Specifications; - Any other data which may be required as per these Specifications or the Conditions of Contract or any other annexures/schedules forming part of the Contract; - Any other item incidental to work which is necessary for complying with the provisions of the Contract; - All temporary works, formwork and false work not included as separate item in the BOQ; - Establishing and running a laboratory with facilities for testing for various items or works as specified in Section 900 and other relevant Sections; - Cost of in-built provisions for Quality Assurance; - Cost of safeguarding the environment; and - Cost of providing “as-built drawings” in original and two sets of prints. 114.3 Portions of road works beyond the limits and/or any other work may be got constructed by the Employer directly through other agencies. Accordingly, other agencies employed by the Employer may be working in the vicinity of the Works being executed by the Contractor. The Contractor shall liaise with such agencies and adjust his construction programme for the completion of work accordingly and no claim or compensation due to any reason whatsoever will be entertained on this account. The Employer will be indemnified by the Contractor for any claims from other agencies on this account. # 115 METHODOLOGY AND SEQUENCE OF WORK ## 115.1 Prior to start of the construction activities at site, the Contractor shall, within 28 days after the date of the agreement unless otherwise stipulated in the Contract, submit to the Engineer for approval, the detailed method statement. The method statement shall be submitted in two parts. ## 115.2 The general part of the method statement shall describe the Contractor’s proposals regarding preliminary works, common facilities and other items that require consideration at the early stage of the contract. The general part shall include information on: ### a) Sources of materials like coarse aggregates and fine aggregates, quantity and quality of materials available in different sources; ### b) Sources of manufactured materials like bitumen, cement, steel reinforcement, pre-stressing strands and bearings etc. He shall also submit samples/test certificates of materials for consideration of the Engineer ; ### c) Locations of the site facilities such as, batching plant, hot mix plant, crushing plant, etc; ### d) Details of facilities available for transportation of men/material and equipment; ### e) Information on procedure to be adopted by the Contractor for prevention and mitigation of negative environmental impact due to construction activities; ### f) Work Zone Traffic Management Plan (WTMP) for Safety and traffic arrangement during construction: ### g) Implementation of activities provided in the Environmental Management Plan; ### h) Any other information required by the Engineer. ## The general part of the QA programme under section 105.3 shall accompany the method statement. ## 115.3 Special part of the method statement along with RFI (Report for Information) format as per IRC:SP: 112 shall be submitted to the Engineer by the Contractor for each important item of work as directed by the Engineer. The statement shall be submitted at least 4 weeks in advance of the commencement of the activity of item of work unless otherwise stipulated in the contract. The statement shall give information on: ### a) Details of the personnel both for execution and quality control of the work; ### b) Equipment deployment with details of the number of units, capacity, standby arrangement; ### c) Sequence of construction and details of temporary or enabling works like diversion, cofferdam, formwork including specialized formwork for superstructure, details of borrow areas, method of construction of embankment, sub-grade and pavement, pile concreting, proprietary processes and products and equipments to be deployed. Wherever required technical literature, design calculations and drawings shall be included in the method statement; ### d) Testing and acceptance procedure including documentation; ### e) The special part of the QA programme under sub-section 105.3 for the particular item of work shall accompany the method statement for the concerned activity. ## The Engineer shall examine and approve the method statement with the required modifications. The modified method statement if required shall be submitted within 14 days of the receipt of the Engineer’s approval. The sole responsibility for adequacy and safety of the method adopted by the Contractor shall rest on the Contractor irrespective of any approval given by the Engineer. ## 115.4 Approval of Proprietary Products/Processes/Systems ## Within 90 days of the signing of agreement, the Contractor shall submit the following information for all proprietary products, process or any other item proposed to be used in the work, for approval of the Engineer. ### a) Name of the manufacturer and name of the product/ process/system along with authenticated copies of the license/collaboration agreement; ### b) General features of the product/process/system; ### c) Details of the product development and development testing; ### d) Acceptance test and criteria; ### e) Installation procedure; ### f) Maintenance procedure and schedule; ### g) Warranty proposal. ## The Engineer may order additional test for the purpose of acceptance. Additional charges for test, if any, for the product/process/system shall be borne by the Contractor. # 116 CRUSHED STONE AGGREGATES Where the terms crushed gravel/shingle, crushed stone, broken stone or stone aggregate appear in any part of the Contract Documents or Drawings issued for work, they refer to crushed gravel/crushed shingle/crushed stone aggregate obtained from integrated crushing plant having appropriate primary crusher, secondary cone crusher, vertical shaft impactor and vibratory screen unless specified otherwise. Stone retained on 4.75 mm sieve shall have at least two faces fractured. # 117 SUPPLY OF QUARRY SAMPLES Raw and processed samples of the mineral aggregates from the approved quarry shall be submitted by the Contractor at his cost. # 118 APPROVAL OF MATERIALS Approval of all sources of material for work shall be obtained in writing from the Engineer before their use on the works. # 119 USE OF SURFACES BY TRAFFIC ## 119.1 Ordinarily, no construction traffic shall be allowed on pavement under construction unless authorized by the Engineer. Even in that case, the load and intensity of construction traffic should be so regulated that no damage is caused to the sub-grade or pavement layers already constructed. Where necessary, service roads shall be constructed for this purpose and the same shall be considered as incidental to the work. ## 119.2 The wheels or the tracks of plant moving over the various pavement courses shall be kept free of deleterious materials. ## 119.3 Bituminous base course shall be kept clean and uncontaminated as long as the same remains uncovered by a wearing course or surface treatment. The only traffic permitted access to the base/binder course shall be that engaged in laying and compacting the wearing course or that engaged on such surface treatment where the base/binder course is to be blinded and/or surface dressed. Should the base/binder course or tack coat on the base/binder course become contaminated, the Contractor shall make good by cleaning it to the satisfaction of the Engineer, and if this is impracticable, by removing the layer and replacing it to Specifications without any extra cost to the employer. ## 119.4. On Dry Lean Concrete sub-base, no heavy commercial vehicles like trucks and buses shall be permitted after its construction. Light vehicles, if unavoidable, may, however, be allowed after 7 days of its construction with prior approval of the Engineer no vehicular traffic shall be allowed on a finished concrete pavement for a period of 28 days of its construction and until the joints are permanently sealed and cured. 120 FIELD LABORATORY # 120.1 Scope The work covers the provision and maintenance of an adequately equipped field laboratory as required for site control on the quality of materials and the works. # 120.2 Description The Contractor shall arrange to provide fully furnished and adequately equipped field laboratory. The field laboratory shall preferably be located adjacent to the site office of the Engineer and provided with amenities like water supply, electric supply etc. as for the site office of the Engineer as described in this Section. The layout and size of the field laboratory shall be as indicated in the drawings or. In case no drawings is furnished, the laboratory shall include space for the storage of samples, equipment, laboratory tables and cupboards, working space for carrying out various laboratory tests, a wash basin, toilet facility and a curing tank for the curing of samples, around 4 m x 2 m x 1 m in size and a fume chamber. Wooden/concrete working table with a working platform area of about 1 m x 10 m shall be provided against the walls. Wooden cupboards above and below the working tables shall be provided to store accessories such as, sample moulds etc. At least 4 racks of slotted angles and M.S. sheets the size 1800 mm x 900 mm x 375 mm and at least 6 stools for laboratory test operators shall also be provided. The items of laboratory equipment shall be provided in the field laboratory depending upon the items to be executed as per Table 100-2. # 120.3 Ownership The field laboratory building and equipment shall be the property of the Contractor. The Employer and the Engineer shall have free access to the laboratory. # 120.4 Maintenance The Contractor shall arrange to maintain the field laboratory in a satisfactory manner until the issue of Taking Over Certificate for the completed work. Maintenance includes all activities described in Section 120.4. Table 100- 2 List of Laboratory Equipment Sl. No; Name of the Item; Number. A; General. 1; Weigh Balances. a); 5 – 20 kg capacity Electronic type – Accuracy 1 gm.; 1 No.. b); 500 gm capacity–Electronic Type Accuracy 0.01 gm; 1 No.. c); Electronic5 kg capacity Accuracy 0.5 gm; 1 No.. d); Platform Balance scale-300 kg capacity; 1 No.. e); Chemical Balance 100 gm capacity-accuracy 0.001 gm. 2; Oven-electrically operated, thermostatically controlled (including thermometer), stainless steel interior from 0°C to 220°C Sensitivity 1°C; 1 No. 3; Sieves : as per IS:460-1962. a); I.S. sieves 450 mm internal dia of sieve sets as per BIS of required sieve sizes complete with lid and pan; 1 set. b); IS sieve 200 mm internal dia (brass frame and steel/or brass wire cloth mesh) consisting of sieve sets of required sieve sizes complete with lid and pan; 2 sets. 4; Sieve shaker capable of aking 200 mm and 450 mm dia sieves-electrically operated with time switch assembly; 1 No. 5; 200 tonnes compression testing machine; 1 No. 6; Stop watches 1/5 sec. accuracy; 1 2 Nos. 7; Glassware comprising beakers, pipettes, dishes, measuring cylinders (100 to 1000 cc capacity) glass rods and funnels, glass thermometers range 0°C to 100°C and metallic thermometers range up to 300°C.; 2 4 No. each. 8; Hot plates 200 mm dia (1500 watt.); 1 No. 9; Enamel trays. a) 600 mm x 450 mm x 50 mm; 2 Nos. b) 450 mm x 300 mm x 40 mm; 2 Nos. c) 300 mm x 250 mm x 40 mm; 2 Nos. d) Circular plates of 250 mm dia; 2 Nos. 10; Water Testing Kit; 1 No. B; FOR SOILS. 1; Water still. 2; Liquid limit device with ASTM grooving tools as per IS:2720; 1 No. 3; Sampling pipettes fitted with pressure and suction inlets, 10 ml. Capacity; 1 set. 4; Compaction apparatus (Proctor) as per IS:2720 (Part 5) (Part 7) complete with collar, base plate and hammer and all other accessories; 1 No. 5; Modified AASHTO Compaction apparatus as per IS:2720 (part 8) 1974 or Heavy CompactionApparatus as per IS complete with collar, base plate hammer and all other accessories; 1 Nos. 6; Sand pouring cylinder with conical funnel and tap and complete as per IS:2720 (part 28 ) 1974 including modern equipment.; 2 Nos. 7; Ennore; As required. 8; Sampling tins with lids 100 mm dia x 75 mm ht. ½ kg capacity and miscellaneous items like moisture tins with lid 50 grams etc.; 4 Nos. 9; Lab CBR testing equipment for conducting CBR testing, load frame with 5 Tonne capacity, electrically operated with speed control as per IS 2720 (Part 16) and consisting of following:; 1 Set. a); CBR moulds 150 mm dia – 175 mm ht. Complete with collar, base plate etc.; 6 No. b); Tripod stands for holding dial gauge holder; 4 Nos. c); CBR plunger with settlement dial gauge holder; 1 No. d); Surcharge weight 147 mm dia 2.5 kg wt. With central hole; 6 Nos. e); Spacers disc 148 mm dia 47.7 mm ht t. With handle; 2 Nos. f); Perforated plate (Brass); 2 Nos. g); Soaking tank for accommodating 6 CBR moulds Proving rings of 1000kg, 2500kg capacity; 2 Nos. h); Proving rings 1000 kg, 2500 kg; 1 No each. i); Dial gauges 25 mm travel – 0.01 mm/division; 2 4 No. 10; Standard penetration test equipment; 1 No. 12; Nuclear moisture Density meter or equivalent; 1 No.. 13; Speedy moisture meter complete with chemicals; 1 2 No. C; FOR BITUMEN AND BITUMINOUS MIXES. 1; Constant temperature bath for accommodating bitumen test specimen, electrically operated, and thermostatically controlled (to accommodate minimum six Specimens); 1 No. 2; Penetrometer automatic type, including adjustable weight arrangement and needles as per IS:1203-1958; 1 No. 3; Soxhlet extraction or centrifuge type apparatus complete with extraction thimbles with solvent and filter paper; 1 No. 4; Bitumen laboratory mixer including required accessories (20 ltrs.); 1 No. 5; Marshall compaction apparatus/ Marshall Stability test apparatus automatically operated as per / ASTM 1559-62 T / complete with accessories (with 180 N Marshall Moulds) “; 1 set. 6; Furol viscometer; 1 No. 7; Ductility meter; 1 No. 8; Softening point (Ring and ball app); 1 No. 9; Distant reading thermometer; –. 10; Rifle box; – 1 No. 11; Automatic Asphalt content Meter; – 1 No. 12; Thin film over oven test apparatus for modified binder either with PMB or CRMB; –. 13; Mastic Asphalt Hardness testing equipment; –. 14; Sand Equivalent test apparatus; 1 set. 15; Core cutting machine suitable for upto 150 mm dia core; 1 set. 16; Thermometers; 4 Nos each. D; FOR CEMENT, CEMENT CONCRETE AND MATERIALS. 1; Water still; 1 No. 2; Vicat needle apparatus for setting time with plungers as per IS:269-1967; 1 No. 3; Moulds. a)150 mm x 300 mm ht. Cylinder with capping component along with the capping set and compound as per IS; As req. b) Cube 150 mm, and 100 mm (each size); As req. 4; Concrete permeability apparatus; -. 5; High frequency mortar cube vibrator for cement testing; -. 6; Concrete mixer power driven, 0.027 cum capacity; -. 7; Variable frequency and amplitude vibrating table size 1 m x 1 m as per the relevant British Standard; -. 8; Flakiness and index test apparatus; 1 No. 9; Aggregate impact test apparatus as per IS:9377/IS:2386 (Part 4) 1963; 1 No. 10; Los-angeles abrasion test apparatus as per IS:2386 (Part 4) 1963; 1 No. 11; Flow table as per IS:712-1973; 1 No. 12; Equipment for slump test; 1 4 No. 13; Equipment for determination of specific gravity or fine and coarse aggregate as per S:2386 (Part 3) 1963; 1 No. 14; Compression and Flexural strength testing achine machine of 200 T capacity with additional dial for flexural testing; 1 No. 15; Core cutting machine with 10 cm diamond cutting edge; 1,. 16; Needle vibrator; 2 Nos. 17; Air entrainment meter; 1 No. 18; 0.5 Cft, 1 C ft, cylinder for checking bulk density of aggregate with tamping rod.; As req. 19; Soundness testing apparatus for cement IS 4031; 1 set. E; FOR CONTROL OF PROFILE AND SURFACE EVENNESS. 1; Total Station with traversing target, having an accuracy of one second.; 1 No. 2; Precision automatic level with micrometer attachment; 1 set. 3; Distomat or equivalent; 1 set. 4; Theodolite – Electronically operated with computerised output attachment; 1 set. 5; Precision staff; 2 sets. 6; 3 meter straight edge and measuring wedge as per IRCSP:16; 1 set. 7; Camber template 2 Lane. a) Crown type cross-section; 1 set. b) Straight run cross-section; 2 sets. 8; Steel tape. a) 5 m long; 2 Nos. b) 10 m long; 2 Nos. c) 20 m long; 2 Nos. d) 30 m long; 2 Nos. e) 50 m long; 1 No. 9; Rougho-meter (Bump Integrator); 1 No ( when required). Note : The items and their numbers listed above in this Section are indicative and shall be decided by the Engineer as per requirements of the Project and modified accordingly. # 120.6 Rate Provision and maintenance of the field laboratory is not a payable item as it is incidental to the work. # 121 SUPPLY OF PROJECT RECORD # 121.1 Scope ## The work covers the supply of digital record of project events in digital format (DVD/Flash Drive) including coloured photographs both in digital format as well as mounted on albums to serve as a permanent record of the work needed for an authentic documentation, as approved by the Engineer. # 121.2 Description ## The Contractor shall provide the following project records in digital format (DVD/Flash Drive) as directed by the Engineer : ### i) Record of work in each work front : It shall cover the status of each workfront before start of work, during various stages of construction and after completion duly including the arrangements made (day & night) for traffic during construction (this shall be need based or as directed by the Engineer); ### ii) Record of quarry sites, plant sites, camp sites including labour camps, haul roads, access roads, etc. on quarterly basis; ### iii) Record of all accidents on project road/ various sites (quarry, plant, camp, etc.) ## The record shall be taken by a professional with a digital camera capable of taking still as well as video images having the facility to record the date and the background commentary. The Contractor shall keep separate discs/drives, one with the Engineer and the other with the Employer and update the data in these discs/drives on monthly basis. Separately, a video (in digital format) of maximum one hour duration covering interesting and novel features of the work duly editing the above master disc/drive shall also be maintained, one copy each kept with the Engineer and the Employer and updated on monthly basis. All recording shall be done in the presence of the Engineer’s Representative who will certify in writing the recording. # 121.3 Measurements for Payment # Supply of two copies of all digital records as above and colour record Photographs both in digital format as well as mounted in the albums of the project shall be measured as one item for the project. # Supply of additional prints of colour record photograph if requested shall be measured in number of additional prints supplied. # The supply of “as-built” drawings in digital format and in hard copies is # incidental to the work and shall not be a payable item. # 121.4 Rate Supply of project record in digital format in two copies (one for the Engineer and the other for the Employer) including video recordings updated on monthly basis throughout the construction period shall be measured as one single item. ---- Appendix 1 (Refer clause 101) LIST OF IRC PUBLICATION REFERRED TO IN SPECIFICATIONS Appendix 2 (Refer clause 103) LIST OF INDIAN AND FOREIGN STANDARDS revised after 2012 Number Designation Title (A) INDIAN STANDARDS IS:73 Paving Bitumen Specification (Second Revision) IS:77-Specification for Linseed Oil;, Boiled, for Paints(Second Revision) IS:102- Ready Mixed Paint, Brushing, Red Lead, Non Setting, Primming IS:104- Ready Mixed Paint, Brushing, Zinc Chrome, Primming (Second Revision) IS:123- Specification for Ready Mixed Paint, Brushing, Finishing, Semi-Gloss. For General Purposes, to Indian Standard Colours (Revised Revision) IS:164- Specification for Ready Mixed Paints for road marking(First Revision) IS:217- Specification for Cutback bitumen (Second Revision) IS:226- Structural Steel (Standard Quality) (Fifth Revision) IS:269- Specification for 33 grade ordinary portland cement (Fourth Revision) IS:278- Galvanized steel barbed wire for Fencing-Specification IS:280- Mild Steel Wire for General Engineering Purposes (Fourth Revision) IS:345- Wood Filter Transparent Liquid IS:356- Ester Gum (Second Revision) IS:383- Specification for Coarse and Fine aggregates from Natural Sources for Concrete (Second Revision) IS:460- Specification for test Sieves: Part III -Methods of Examination of Apertures of Test Sieves(Third Revision) IS:503 Alloy Austenitic Manganese Steel Castings IS:516 -1959 Methods of Test for Strength of Concrete IS:712- Specifications for Building Limes(Third Revision) IS:736-6 Wrought Aluminum and Aluminum Alloys, Plates for General Engineering Purposes (Fourth Revision) IS:783- Code of Practice for Laying of Concrete Pipes (First Revision) IS:784- Prestressed Concrete Pipes (Including Fittings) –Specifications (Second Revision) IS:814- Covered Electrodes for Manual Metal Arc Welding of Carbon and Carbon Manganese Steel (Fifth Revision) IS:816- Coe of Practice for Use of Metal Arc Welding for General Construction in Mild Steel (First Revision) IS:817- Code of Practice for Training and Testing of Metal Arc Welders(Revised Revision) IS:822Code of Procedure for Inspection of Welds IS:877- Methods of Sampling and Test for Activated Carbons, Powered and Granular (Second Revision) IS:919- (Part-1) ISO Systems of Limits and Fits Part 1 Bases of Tolerance, Deviation and Fits (Second Revision) IS:919- (Part-2 ISO Systems of Limits and Fits Part 2 Tables of Standard Tolerance Grades and Limit Deviations for Holes and Shafts (First Revision) IS:961- Structural Steel (High Tensile) (Second Revision) IS:1077- Common BURNT CLAY BUILDING BRICKS (Fifth Revision) IS:1079- Hot Rolled Carbon Steel Sheets and Strips-Specifications (Sixth Revision) IS:1124- Method of Test for Water Absorption, Apparent Specific Gravity and Porosity of Natural Building Stones (First Revision) IS:1129-2 Recommendation for Dressing of Natural Building Stones (First Revision) IS:1148-2 Hot Rolled Rivet Bars(upto 40 mm dia) for Structural Purposes (Third Revision) IS:1149-Hot Tensile Steel Rivet Bars for Structural Purposes (Third Revision) IS:1182- Recommended Practice for Radiographic Examination of Fusion Welded Butt Joints in Steel Plates(Second Revision) IS:1199- Method of Sampling and Analysis of Concrete IS:1203- Determination of Penetration IS:1205- Determination of Softening Point IS:1212- Determination of Loss on Heating IS:1216- Determination of Solubility in Carbon DisulphideTrichlorothylene IS:1217- Determination of Mineral Matter IS:1239:(Part 1)- Steel Tubes, Tubulars and Other Wrought Steel Fittings-Specification: Part 1 Steel Tubes IS:1363- Hexagonal Head Bolts, Screws and Nuts of Product Grade ‘C’(Part 1,2 & 3) IS:1364- Hexagonal Head Bolts, Screws and Nuts of Product Grade ‘A’ & ‘B’(Part 1,2 &3) IS:1365- Slotted Countersunk, Flat Head Screws(Common Head Style)-Product Grade A (Fourth Revision) IS:1367- Technical supply conditions for Threaded Steel Fasteners IS:1393- Code of Practice for Training and Testing of Oxy Acetylene Welders IS:1477- Code of Practice for Painting of Ferrous Metals in Buildings (Parts 1 & 2) IS:1489- SPECIFICATION FOR Portland pozzolana Cement(Third Revision)(Part 1) Fly ash Based (Third Revision) (Part 2)Calcined Clay Based (Third Revision) IS:1498- Classification and Identification of Soils for General Engineering Purposes (First Revision) IS:1514- Methods of Sampling and Test for Quick Lime and Hydrated Lime (First Revision) IS:1597- Construction of Stone Masonry - Code of Practice - Part 1 - Rubble (Part 1) Stone Masonry (First Revision) IS:1732- Dimensions for Round and Square Steel Bars for Structural and General Engineering Purposes (Second Revision) IS:1745- Specification for Petroleum Hydrocarbon Solvent(Second Revision) IS:1785- (Part-1) Specification for Plain, Hard Drawn Steel Wire for Prestressed Concrete: Part 1 Cold Drawn Stress Relieved Wire(Second Revision) IS:1785- (Part-II) Specification for Plain, Hard Drawn Steel Wire for Prestressed Concrete: Part 2 As Drawn Wire(First Revision) IS:1786- High Strength Deformed Steel Bars and Wires for Concrete Reinforcement (Third Revision) IS:1834- Specification for Hot Applied Sealing Compounds for Joints in Concrete (First Revision) IS:1838- (Part 1) Specification for Preformed Filler for Expansion Joint in Concrete Pavement and Structures (Non-Extruding and Resilient Type):Part 1 Bitumen Impregnated Fibre (First Revision) IS:1838- (Part 2) Specification for Preformed Filler for Expansion Joint in Concrete Pavement and Structures (Non-Extruding and Resilient Type):Part 2 CNSL Aldehyde Resin and Coconut Pith IS:1852- Rolling and Cutting Tolerances for Hot Rolled Steel Products(Fourth Revision) IS:1875- Carbon Steel Billets, Blooms, Slabs and Bars for Forgings (Fifth Revision) IS:1888- Method of Load Test on Soils(Second Revision) IS:1892- Code of Practice for Sub Surface Investigation for Foundations (First Revision) IS:1966-2003 Laboratory Glassware-Straight Bore Glass Stopcocks for General Purposes(First Revision) IS:1977- Low Tensile Structural Steels –Specifications(Third Revision) IS:1995- Overall Internal Heights for Lathe Tool Posts (First Revision) IS:2004- Carbon Steel Forgings for General Engineering Purposes (Third Revision) IS:2016- Specification for Plain Washers (First Revision) IS:2062- Hot Rolled Low, Medium and High Tensile Structural Steel (Sixth Revision) IS:2090- Specifications for High Tensile Steel Bars used in Prestressed Concrete(First Revision) IS:2116- Specification for Sand for Masonry Mortars (First Revision) IS:2131- Method for Standard Penetration Test for Soils (First Revision) IS:2132- Code of Practice for Thin Walled Tube Sampling of Soils (Second Revision) IS:2185- (Part-1) Concrete Masonry Units-Specification: Part 1-Hollow and Solid Concrete Blocks IS:2250-1981 Code of Practice for Preparation and Use of Masonry Mortars (First Revision) IS:2269- Hexagon Socket Head Cap Screws (Fifth Revision) IS:2339- Aluminum Paint for General Purposes in Dual Container IS:2386- Methods of Test for Aggregates for Concrete (Part 1) Particle size and shape (Part 2) Estimation of Deleterious Materials and Organic Impurities (Part 3) Specific Gravity, Density, Voids, Absorption and Bulking (Part 4) Mechanical Properties (Part 5) Soundness (Part 6) Measuring Mortar Making Properties of Fine Aggregates (Part 7) Alkali Aggregate Reactivity Test (Part 8) Petrographic examination IS:2502- Code of Practice for Bending and Fixing of Bars for Concrete Reinforcement IS:2506- General requirements for Screed Board Concrete Vibrators(First Revision) IS:2514- Specification for Concrete Vibrating Tables IS:2720 Methods of Test for Soils (Part 1)- Preparation of Dry Soil Samples for Various Tests (Second Revision) (Part 2)- Determination of Water Content (Second Revision) (Part 4)- Grain Size Analysis (Second Revision) (Part 5)- Determination of Liquid and Plastic Limits (Second Revision) (Part 8)- Determination of Water Content Dry Density Relation using Heavy Compaction (Second Revision) (Part 16)- Laboratory Determination of CBR (Part 27)- Determination of Total Soluble Sulphates(First Revision) (Part 28)- Determination of Dry Density of Soils in-place by the Sand Replacement Method(First Revision) (Part-37)- Determination of Sand Equivalent Values of Soils and Fine Aggregates (Part 40)- Determination of Free Swell Index of Soils IS:2751- Code of Practice for Welding of Mild Steel Plain and Deformed Bars for Reinforced Concrete Construction (First Revision) IS:2911 Code of Practice for Design and Construction of Pile Foundations (Part-1)(Sec-3) - Concrete Piles, Section 3 Driven Precast Concrete IS: 2925- Specification for Industrial Safety Helmets (Second Revision) IS:3025:(Part 17)- Methods of Sampling and Test (Physical And Chemical) for Water and Wastewater: Part 17 Non-Filterable Residue (Total Suspected Solids) (First Revision) IS:3025:(Part 18)- Methods of Sampling and Test (Physical and Chemical) for Water and Waste Water- Part 18 Volatile and Fixed Residue (Total Filterable and Non-Filterable) (First Revision) IS:3025:(Part 22)- Methods of Sampling and Test (Physical and Chemical) for Water and Waste Water- Part 22 : Acidity (First Revision) IS:3025: (Part 23)- Methods of Sampling and Test (Physical and Chemical) for Water and Waste Water- Part 23: Alkalinity (First Revision) IS:3025: (Part 28)- Methods of Sampling and Test (Physical and Chemical) for Water and Waste Water- Part 28: Sulphite (First Revision) IS:3025 (Part 32)- Methods of Sampling and Test (Physical and Chemical) for Water and Waste Water- Part 32: Chloride (First Revision) IS:3073- Assessment of Surface Roughness IS:3138 Specifications for Hexagonal Bolts and Nuts IS:3400 (Part 3)- Methods of Test for Vulcanized Rubbers: Part 3 Abrasion Resistance Using a Rotating Cylindrical Drum Device (First Revision) IS:3400 (Part 2)- Methods of Test for Vulcanized Rubbers: Part 2 Rubber Vulcanized or Thermoplastic-Determination of Hardness (Hardness between 10 IRHD and 100 IRHD) (Third Revision) IS:3400 (Part 4)- Methods of Test for Vulcanized Rubbers: Part 4 Accelerated ageing (Second Revision) IS:3400 (Part 10)- Methods of Test for Vulcanized Rubbers: Part 10 Compression Set at Constant Strain (First Revision) IS:3400 (Part 14)- Methods of Test for Vulcanized Rubbers: Part 14 Adhesion of Rubber to Metal (First Revision) IS:3400 (Part 20)- Methods of Test for Vulcanized Rubbers: Part 20 Resistance to Ozone Cracking-Static Strain Test (First Revision) IS:3589- Steel Pipes for Water and Sewage (168.3 to 2540 mm Ouside Diameter)- Specification (Third Revision) IS:3613 Acceptance Tests for Wire Flux Combination for Submerged Arc Welding 2010 Code of Practice for Liquid Penetrant Flaw Detection (Second Revision) IS:3764- 2012 Code of Safety for Excavation Work (First Revision) IS:3784- 2010 Textiles –Cloth, Cotton Khadi Bleached for General Purposes- Specification (First Revision) IS:3812:2013 Specification for Fly Ash for Use as Pozzolana and Admixture IS:4031:(Part 5)- Methods of Physical Tests for Hydraulic Cement: Part 5 Determination of Initial and Final Setting Times (First Revision) IS:4078- Code of Practice for Indexing and Storage of Drill Cores (First Revision) IS:4081- Safety Code for Blasting and Related Driling Operations (First Revision) IS:4138- Safety Code for Working in Compressed Air(First Revision) IS:4260- Recommended Practice for Ultrasonic Testing of Butt Welds in Ferritic Steel (Second Revision) IS:4434- Code of Practice for In-Situ Vane Shear Test for Soils (First Revision) IS:4453- Subsurface Exploration by Pits, Trenches, Drifts and Shafts-Code of Practice (Second Revision) IS:4656- Specification for Form Vibrators for Concrete IS:4826- Hot Dipped Galvanised Coating on Round Steel Wires (First Revision) IS:4853- Recommended Practice for Radiographic Inspection of Fusion Welded Butt Joints in Steel Pipes (First Revision) IS:4923 Hollow Steel Sections for Structural Use(Second Revision) IS:4925 Concrete Batching and Mixing Plant-Specification (First Revision) IS:4926 Ready Mixed Concrete – Code of Practice(Second Revision) IS:4968- Method for Subsurface Sounding for Soils (Part-1,2 &3) IS:4984- Specification for High Density Polyethylene Pipes for Potable Water Supplies (Fourth Revision) IS:5334 2014 Magnetic Particle Flaw Detection of Welds-Code of Practice (Second Revision) IS:5435- General Requirements For Cold Bituminous Macadam Mixing Plants (First Revision) IS:5640 Method for Determining the Aggregate Impact Value of Soft Coarse Aggregate IS:6003- Specification for indented Wire for Prestressed Concrete (First Revision) IS:6006- Specification for Uncoated Stress Relieved Strand for Prestressed Concrete (First Revision) IS:6241- Methods of test for Determination of Stripping Value of Road Aggregates IS:6603- Stainless Steel Bars and Flats - Specification (First Revision) IS:6610- Specification for Heavy Washers for Steel Structures IS:6639- Specification for Hexagonal Bots for Steel Structures IS:6761- Fasteners-Countersunk Head Screws with Hexagonal Socket- Specification(First Revision) IS:6909- Stainless Steel Plate, Sheet and Strip (First Revision) IS:6925-1973 Methods forTest for Determination of Water soluble chlorides in Concrete Admixtures IS:7205-1974 Safety Code for Erection of Structural Steelwork IS:7269-1974 Numbering of Aircraft Engines, Engine Cylinders, Combustion Chambers and Direction of Rotation of Engines and Propellers IS:7273-1974 Method of Testing Fusion Welded Joints in Aluminium andAluminium Alloys IS:7292-1974 Code of Practice for In-Situ Determination of Rock Properties by Flat Jack IS:7293-1974 Safety Code for Working with Construction Machinery IS:7307 (Part 1)-1974 Approval Tests for Welding Procedures Part-1 Fusion Welding of Steel IS:7308-1999 Non Coniferous Logs-Specification (First Revision) IS:7310(Part-1)- Approval Tests for Welders Working to Approved Welding Procedures-Part1: Fusion Welding of Steel IS:7317- Code of Practice for Uniaxial Jacking Test for Deformation Modulus of Frock (First Revision) IS:7573-1975 Hockey Shoes IS:7746-1991 Code of Practice for In-Situ Shear Test on Rock (First Revision) IS:7966-1976 Specification for Dental Modelling Wax IS:8500-1991 Structural Steel- Micro alloyed (Medium and High Strength Qualities)- Specification IS:8812- (Part1) Methods for Chemical Analysis of Hard Solders for Jointing Aluminium and Aluminium Alloys Part 1 Determination of Silver, Copper, Zinc, Antimony, Iron and Bismuth IS:8812- (Part2) Methods for Chemical Analysis of Hard Solders for Jointing Aluminium and Aluminium Alloys Part 2 Determination of Aluminium IS:9000-2005 Quality Management Systems-Fundamentals and Vocabulary(Third Revision) IS:9012- 2009 Recommended Practice for Shotcreting IS:9437-2008 Specification for Fixed Precision Resistors IS:9565-1995 Acceptance Standards for Ultrasonic Inspection of Steel Castings (Second Revision) IS:9595-1996 Metal Arc Welding of Carbon nd Carbon Manganese Steels – Recommendations (First Revision) IS:9862- Ready Mixed Paint, Brushing, Bituminous, Black, Lead Free, Acid, Alkali, Water and Chlorine Resisting IS:10433(Part 1)-1983 Specification for Male Stud Tee Body (Stud Run) for Oil –Hydraulic Coupling- Part-1: Made form Forging IS:11587-1986 Structural Weather Resistant Steels IS:12063-1987 Classification of Degrees of Protection provided by Enclosures of Electrical Equipment IS:12089-1987 Specification for granulated Slag for Manufacture of Portland Slag Cement IS:12119-1987 General Requirements for Pan Mixers for Concrete 1S:12269 Specification for 53 Grade Ordinary Portland Cement IS:12330- Specification for Sulphate Resisting Portland Cement IS:12594-1988 Hot Dip Zinc Coating on Structural Steel Bars for Concrete Reinforcement- Specification IS:13162(Part-4)-1992 Geotextiles- Methods of Test-Part 4: Determination of Puncture Resistance by Falling Cone Method IS:13162(Part-5)-1992 Withdrawn Geotextiles- Methods of Test-Part 5: Determination of Tensile Properties using a Wide Width Strip IS:13360- Plastics-Methods of Testing IS:13620- Fusion Boned Epoxy Coated Reinforcing Bars-Specification IS:13757- Burnt Clay Fly Ash Building Bricks- Specifications IS:13759-1993 Polyurethane Primer Zinc Phosphate (Two Pack) for Exterior Painting of Railway Coaches IS:14293-1995 Geotextiles- Method of Test for Trapezoid Tearing Strength IS:14294- Geotextiles-Method for Determination of Apparent Opening Size by Dry Sieving Technique IS:14324-1995 Geotextiles –Methods of Test for Determination of Water Permeability- Permittivity IS:14589-1999 Zinc Priming Paint, Epoxy Based Two Pack-Specification IS:14700 (Part 6 Sec 3) Electromagnetic Compatibility (EMC)- Part 6 Generic Standards- Emission Standards for Residential, Commercial and Light Industrial Environments IS:14925-2001 Epoxy Resin for Paints-Specification IS:15284 (Part 1)-2003 Design and Construction for Ground Improvement-Guidelines: Part 1 Stone Columns (Part 2)-2004 Design and Construction for Ground Improvement-Guidelines: Part 2 Pre-consolidation using Vertical Drains IS:15388- Silica Fume-Specification IS:15462- Polymer and Rubber Modified Bitumen-Specification IS:15809- High Visibility Warning Clothes-Specification IS:SP:23-1982 Handbook on concrete mixes (based on Indian Standards) (B) FOREIGN STANDARDS ASTM E8 Standard Test Methods for Tension Testing of Metallic Materials ### ASTM–E 11-2020 Standard Specification for Woven Wire Test Sieve Cloth and Test Sieves ASTM 36 - 1 Standard Specification for Carbon Structural Steel ASTM D-297- Standard Test Methods for Rubber Products-Chemical Analysis ASTM-C 309-81 Standard Specification for Liquid Membrane-FormingCompounds for Curing Concrete ASTM-C 309-81 Standard Practice Standard Specification for Architectural Flat Glass Clad Polycarbonate ### ASTM 393 Standard Test Method for Core Shear Properties of Sandwich Constructions by Beam Flexure ### ASTM C393 Test Method for Core Shear Properties of Sandwich ### Constructions by Beam Flexure ASTM D 412 Standard Test Methods for Vulcanized Rubber and Thermoplastic Elastomers-Tension ASTM -D 471- Standard Test Method for Rubber Property-Effect of Liquids ASTM D 573- Standard Test Method for Rubber-Deterioration in an Air Oven. ASTM -711- Standard Specification for Steel Forging Stock ### ASTM A 732-2020 Standard Specification for Castings, Investment, Carbon and Low . Alloy Steel for General Application, and Cobalt Alloy for High Strength at Elevated Temperatures # ASTM 788- Standard Specification for Steel Forgings, General # Requirements ASTM E 809 Standard Practice for Measuring Photometric Characteristics of Retro-reflectors ASTM E 810 - Standard Test Method for Coefficient of Retro-reflection of Retro-reflective Sheeting Utilizing the Coplanar Geometry. ASTM D 903- Standard Test Method for Peel or Stripping Strength of Adhesive Bonds ASTM 968- Standard Test Methods for Abrasion Resistance of Organic Coatings by Falling Abrasive ASTM 1149- Standard Test Methods for Rubber Deterioration-Cracking in an Ozone Controlled Environment ASTM 1264- Standard Classification for Acoustical Ceiling Products ASTM D1475- Standard Test Method for Density of Liquid Coatings, Inks, and l Related Products ASTM D 1505- Standard Test Method for Density of Plastics by the Density-Gradient Technique ASTM F1553- Standard Guide for Specifying Chain Link Fence ASTM 1644- Standard Test Methods for Non-volatile Content of Varnishes ASTM ASTM D 1693- Standard Test Method for Environmental Stress-Cracking of Ethylene Plastics 1647- Standard Test Methods for Organic Matter Content of Athletic Field Rootzone Mixes ASTM D 2041 Standard Test Methods for Quantitative Extraction of Bitumen From Bituminous Paving Mixtures ASTM D2240- Standard Test Method for Rubber Property-Durometer Hardness ASTM D2628- Standard Specification for Preformed Polychloroprene Elastomeric Joint Seals for Concrete Pavements. ASTM D2990- Standard Test Methods for Tensile, Compressive, and Flexural Creep and Creep-Rupture of Plastics ASTM D-3776- Standard Test Methods for Mass Per Unit Area (Weight) of Fabric ASTM D3786- Standard Test Method for Bursting Strength of Textile Fabrics Diaphragm Bursting Strength Tester Method ASTM D4060- Standard Test Method for Abrasion Resistance of Organic Coatings by the Taber Abrase ASTM D 4280- Standard Specification for Extended Life Type, Non-plowable, Raised Retro-reflective Pavement Markers ASTM D 4491 - Standard Test Methods for Water Permeability of Geotextiles by Permittivity ASTM D4533 - Standard Test Method for Trapezoid Tearing Strength of Geotextiles ASTM D-4595- Standard Test Method for Tensile Properties of Geotextiles by the Wide-Width Strip Method ASTM D -4632- Standard Test Method for Grab Breaking Load and Elongation of Geotextiles ASTM D-4716 - Standard Test Method for Determining the (In-plane) Flow Rate per Unit Width and Hydraulic Transmissivity of a Geosynthetic Using a Constant Head ASTM D-4751- Standard Test Method for Determining Apparent Opening Size of a Geotextile ASTM D:4873- Standard Guide for Identification, Storage, and Handling of Geosynthetic Rolls and Samples ASTM D-4956 - 1 Standard Specification for Retro-reflective Sheeting for Traffic Control ASTM D- 5035- Standard Test Method for Breaking Force and Elongation of Textile Fabrics (Strip Method) ASTM 5199- Standard Test Method for Measuring the Nominal Thickness of Geosynthetics # ASTM D-5261 Standard Test Method for Measuring Mass per Unit Area of Geotextiles ASTM D:5581- Standard Test Method for Resistance to Plastic Flow of Bituminous Mixtures Using Marshall Apparatus (6 inch-Diameter Specimen) ### ASTM-C :5893-10 Specification for Cold Applied, Single Component ASTM D 6241-1971 Standard Test Method for the Static Puncture Strength of Geotextiles and Geotextile-Related Products using a 50-mm Probe ASTM D 6390- Standard Test Method for Determination of Draindown Characteristics in Uncompacted Asphalt Mixtures ASTM D 6637- 1 Standard Test Method for Determining Tensile Properties of Geogrids by the Single or Multi-Rib Tensile Method ASTM D-6525- Standard Test Method for Measuring Nominal Thickness of Rolled Erosion Control Products # AASHTO M P8 Standard Specification for Designing Stone Matrix Asphalt # (SMA) AASHTO PP41 Standard Practice for Designing Stone Matrix Asphalt (SMA) AASHTO T-245 Standard Method of Test for Resistance to Plastic Flow of Bituminous Mixtures using Marshall Apparatus AASHTO M-249 Specifications (White & Yellow Reflective Thermo-(Solid Form) AASHTO T 283 – Standard Method of Test for Resistance of Compacted Hot Mix Asphalt (HMA) to Moisture-Induced Damage BS EN 169:2002 Personal Eye-Protection Filters for welding and related Techniques. Transmittance Requirements and Recommended Use Plastics. BS EN ISO 175:2010 Methods of Test for the Determination of the Effects of Immersion in Liquid Chemicals BS EN 397:2012 Industrial Safety Helmets BS EN 471 :2003 High-Visibility Warning Clothing for Professional Use-Test methods and requirements BS:812-1975 Part-114 Testing aggregates method for determination of the polished stone value BS 873-4:1973 Specification for the Construction of Road Traffic Signs and Internally Illuminated Bollards & Road Studs BS:1047 - Specification for Air-cooled Blast Furnace Slag Aggregate for Use in Construction (Inactive) BS:1377 - Methods of Test for Soils for Civil Engineering Purposes BS 1449-2:1983 Specification for Stainless and Heat-Resisting Steel Plate, Sheet and Strip BS EN ISO 3262-12:2001 Extenders for Paints. Specifications and Methods of Test. Muscovite-Type Mica BS 4482 - Reinforcement of Concrete BS 4483- Specification for Steel Fabric for the Reinforcement of Concrete. BS:5212 Specification for Cold Poured Joint Sealants for Concrete Pavements BS 6088:81 Specification for Solid Glass Beads for Use with Road Marking Compounds and for other Industrial Uses BS:7542- Method of Test for Curing Compounds for Concrete BS EN IS0 9863-1 :2005 Geosynthetics: Determination of Thickness at Specified Pressures. Single Layers BS EN ISO 9864:2005 Geosynthetics: Test Method for the Determination of Mass per Unit Area of Geotextiles and Geotextile-Related Products BS EN ISO 10319:2008 Geosynthetics: Wide-width Tensile Test (Withdrawn) ### BS EN ISO 12236:2006 Geosynthetics: Static Puncture Test (CBR Test) #### BS EN ISO 12966-2:2011 7 Animal and Vegetable Fats and Oils. Gas Chromatography #### of Fatty Acid Methyl Esters. Preparation of Methyl Esters #### of Fatty Acides BS EN ISO 12958:2010 Geotextiles and Geotextile-related products. Determination of Water Flow Capacity in their Plane BS EN 13422:2004 Vertical Road Signs. Portable Deformable Warning Devices and Delineators. Portable Road Traffic Signs. Cones and Cylinders BS EN 50128:2011 Railway Applications. Communication, Signalling and Processing Systems. Software for Railway Control and Protection Systems | Section 100 Approved by MORTH dt. 11.03.2025 INTRODUCTION 101.1 These Specifications shall apply to all such road and bridge Works as are required to be executed under the Contract or otherwise directed by the Engineer-in-Charge (hereinafter referred to as the Engineer). In every case, the work shall be carried out to the satisfaction of the Engineer and conform to the location, lines, dimensions, grades and cross- sections shown on the drawings or as decided by the Engineer. The quality of materials, processing of materials as may be needed at the site, salient features of the construction work and quality of finished work, measures for safety of workers and public, traffic arrangements during execution shall comply with the requirements set forth in succeeding sections. Where the drawings and Specifications describe a portion of the work only in general terms, and not in complete detail, it shall be understood that only the sound engineering practices is to prevail, materials and workmanship of the best quality are to be employed and the instructions of the Engineer are to be fully complied with 101.2 A list of Indian Roads Congress (IRC) Specifications and Recommended Codes of Practice which have been referred in these Specifications is given at Appendix-1. The latest edition of all Specifications/Standards/Codes of IRC issued 60 days before final date of submission of the tender or as specified in the contract shall be adopted. 101.3 In case of any conflict or inconsistency in the provisions of the applicable Specifications/Standards/Codes of IRC, latest provisions contained in the applicable Specifications/Standard/Codes of IRC shall apply. 102. ABBREVIATION AND DEFINITIONS - Abbreviations: The following abbreviations, when used in the Contract, represent the full text shown: - AASHTO :American Association of State Highway and Transportation Officials - ABS :Acrylonitrile Butadiene Styrene - ACM :Aluminium Composite Material - AIV :Aggregate Impact Value - AIS :Automotive Industry Standard - AMG :Automated Machine Guidance - AQI :Air Quality Index - ASA :Acrylic Styrene Acrylonitrile - ASI :Acceleration Severity Index - ASTM :American Society for Testing and Materials - AVCC :Automatic Vehicle Classification & Counting - AVC :Automatic Vehicle Classification - BC :Bituminous Concrete, - BIS :Bureau of Indian Standards - BOQ :Bill of Quantities - BPT :British Pendulum Tester - BOT :Build, Operate and Transfer - BS :British Standard published by the British Standards Institution - CBR :California Bearing Ratio - CCTV :Close Circuit Television - CVPD :Commercial Vehicles per Day - C&D :Construction & Demolition - CE : Certificate of European Compliance - C.I.L : Coefficient of Luminous Intensity - CMS : Changeable Message Sign - CRMB : Crumb Rubber Modified Bitumen - DBM : Dense Bituminous Macadam - DL : Drainage Layer - DLC : Dry Lean Concrete - DMS : Dynamic Message Sign - ECB : Emergency Call Box - EI : Elongation Index - EN : European Norm - EPC : Engineering, Procurement and Construction - FHWA : Federal Highway Administration - FI : Flakiness Index, - F/B : Fine to Binder Ratio - FAM : Fault and Alarm Monitoring - GUI : Graphic User Interface - GSB : Granular Sub-Base - Gmm : Theoretical Maximum Specific Gravity - JMF : Job Mix Formula - GI : Galvanized Iron - Gmb : Actual Bulk Specific Gravity - HIP : High Intensity Prismatic Grade Sheeting - HMA : Hot Mix Asphalt - HIPS : Hi-impact polystyrene - ITS : Indirect Tensile Strength - IRC : Indian Roads Congress - IS : Indian Standard (published by the Bureau of Indian Standards) - ISO : International Organization for Standardization - IMD : Indian Metrological Department - JMF : Job Mix Formula - LL : Liquid Limit - LDPE : Low Density Polyethylene - LED : Light Emitting Diode - LLDP : Linear Low-Density Polyethylene - LDR : Light Dependent Resistor - LiDAR : Light Detection and Ranging - MTTR : Mean-Time-to-Repair - MASH : (American) Manual for Assessing Safety Hardware - MIT : Magnetic Integrating Tool - MPI : Magnetic Pulse Induction - MARV : Minimum Average Roll Value - MORTH : Ministry of Road Transport and Highways - N : Newton - NCHRP : National co-operative Highway Research Programme - NMS : Network Management System - NHAI : National Highways Authority of India - OFC : Optical Fibre Cable - OPC : Ordinary Portland Cement, - OIV : Occupant Impact Velocity - ORDA : Occupant Ride Down Acceleration - PL : Plasticity Limit - PQC : Pavement Quality Concrete - PPC : Pozzolana Portland Cement - PPP : Public Private Partnership - PSC : Portland Slag Cement - PMB : Polymer Modified Bitumen - PCC : Profile Correction Course - PI : Plasticity Index - PVC : Poly Vinyl Chloride - PVD : Prefabricated Vertical Drain - PMC : Pre-Mix Carpet - PC : Personal Computer - Pa : Percentage Air Voids - PSA : Public Service Announcements - PUC : Pollution Under Control - QA : Quality Assurance - ROB : Road over Bridge - RUB : Road under Bridge - RMP : Reflective Pavement Marker - RCC : Reinforced Cement Concrete - RRL : Road Research Laboratory - SAL : Stress absorbing Layer - SAM : Stress absorbing Membrane - SI : International System of Units - SMA : Stone Matrix Asphalt - SAM : Stress absorbing Membrane - SAMI : Stress absorbing Membrane Interlayer - TSR : Tensile Stress Ratio - Tult. : Tensile Strength - TFT : Thin Film Transistor - THIV : Theoretical Head Impact Velocity - UCS : Unconfined Compressive Strength - UV : Ultra Violet Light - VG : Viscosity grade - VFB : Voids Filled with Bitumen - VMA : Voids in Mineral Aggregate - VCA : Voids in Coarse Aggregates - VMS : Variable Message Signs - WBM : : Water Bound Macadam - WMM: : Wet Mix Macadam - W/C Raito : Water Cement Ratio - Definitions: The terms, when used in the Contract have the meaning described in Annexure I. The words like Contract, Contractor, Engineer (synonymous with Engineer-in-Charge), Drawings, Employer, Government, Works, and Work Site used in these Specifications shall be considered to have the meaning as understood from the definitions of these terms as given in the respective Contract. The various elements in the Typical Cross-sections of a road referred to in these specifications are shown in the cross-sections in Figure 100-1, 100-2, 100-3, 100-4, 100-5 and 100-6. These are illustrative only. - SCOPE OF WORK - The work, to be carried out under the Contract shall consist of the various items as generally described in the Contract / Concession Agreement. - Programme of Works and Temporary Works - Programme of Works: In accordance with relevant clauses of the Conditions of Contract and prior to commencement of permanent Works, the Contractor shall submit a fully detailed and time related programme showing the order of procedure and method in which the Contractor propose to carry out the Works. The programme shall be so detailed as to show: The order of work. Planned rate of progress. Quantity and type of equipment proposed. Details of methods to be employed. Details of Temporary Works. The programme shall not be in form of a bar chart only, but it shall show the critical path of activities. An initial construction schedule using a CPM shall be prepared, that is computer- generated. An initial construction schedule shall be submitted, that includes all activities, contractual requirements (such as construction or traffic staging), contract restrictions, and subcontract work. The Works shall be carried out so as to achieve a continuous and consecutive output of fully completed road. The order of execution of the Works shall be subject to reasonable adjustment as requested by the Engineer. - Temporary Works: The Contractor shall supply to the Engineer with drawings for approval showing the layout and general arrangement of all Temporary Works he proposes to construct for the purpose of the Contract including, but not limited to: - Camps, including accommodation for staff and labour. - Offices - Laboratory - Workshops - Stores - Aggregate crushing plants - Bitumen storage facilities, etc. - Concrete manufacturing plants - Precast concrete yards - Temporary river crossings - Temporary bypass and access roads. - Temporary structures for supporting cast cast in situ construction and/or erection/launching of super structure of permanent bridges/structures No separate payment will be made for Temporary Works, the relevant cost being incidental to items in bill of quantities/scope of work. - Conformity with Drawings/Allowable Deviations All Works performed and all materials furnished shall be in conformity with the lines, grades, typical sections, dimensions, material requirements, and tolerances shown in the drawings or as indicated in the Specifications. - The Works to be performed shall also include all general Works preparatory to the construction of roads, bridges, highway, canal crossings, drainage and all other related Works. The Works shall include work of any kind necessary for the due and satisfactory construction, completion and maintenance of Works to the intent and meaning of the drawings and these Specifications and further as directed by the Engineer from time to time. The scope of work shall include compliance by the Contractor with all Conditions of Contract, whether specifically mentioned or not in the various Sections of these Specifications, all materials, apparatus, plant, equipment, tools, fuel, water, shuttering, timbering, transport, offices, stores, Workshop, staff, labour and the provision of proper and sufficient protective Works, diversions, temporary fencing and lighting required for the completion of work. Contractor shall establish his field office in the camp site and establish field laboratory at his own cost. It shall include all Works related to safety of road user. It shall also include safety of workers at construction site, medical staff, first-aid equipment, suitable drinking water facility, accommodation for the staff , security men and workmen with adequate sanitary arrangements all types of Insurances to the staff, engineers, security men and workers, the payment of all wages, salaries, fees, royalties, duties or other charges arising out of the execution of Works and the regular clearance of debris, rubbish, reinstatement and clearing- up of the site as may be required on completion of Works, safety of the public and protection of the Works and adjoining land/structures. - The Contractor shall ensure that all actions are taken to build in Quality Assurance (QA) in every stage of project implementation, the planning, management and execution of Works. The quality assurance shall cover all stages of work such as setting out, selection of materials, selection of construction methods, selection of equipment and plant, deployment of personnel and supervisory staff, quality control testing, etc. The QA programme shall cover the details as per IRC:SP:112. These shall broadly cover quality assurance aspects of all services rendered, all items to be supplied and all activities to be performed under the Contract Agreement including temporary structures and equipment which will influence the quality of the completed Works or the progress of the contract. As a minimum Quality Assurance programme it shall cover the following: - Quality Policy of the Contractor - Organisation and management responsibility, Responsibility matrix among the Contractor’s key personnel - Document and data control, - Work Zone Traffic Management Plan (WTMP), - Construction programme, - Method statement, - Process control, of material, machinery, laboratory and testing - Working, inspection, testing and documention, procedures, - Arrangement for smooth and safe traffic flow during construction and maintenance, - Control and documentation of purchasing and handling of materials, - Maintenance of records for non-conformity and timely corrective actions, - Internal quality audit, - Training of staff, - Environment Management Plan (EMP). The QA plan shall be submitted to the Engineer for approval, not later than 28 days from the date of signing of the Contract Agreement. The work of building in quality assurance shall be deemed to be covered in the scope of the work. - The Contractor shall furnish, at least 28 days in advance, unless otherwise stipulated in the contract, the programme of commencement of each item of work, including the method statement deployment of plant and equipment for the Works included in the contract and any other work for which the Engineer may demand the method statement including periodic calibration of equipment and machinery as per IRC: SP:112 or relevant codes. The Contractor shall provide all information to the satisfaction of the Engineer to ensure its adequacy. The sole responsibility for the safety and adequacy of the methods adopted by the Contractor will, however, rest on the Contractor, irrespective of any approval given by the Engineer. - Inspection of Materials before Incorporation All materials shall be inspected, tested and accepted by the Engineer as per these Specifications, before incorporation in the work. The frequencies and methods of sampling and testing materials, including those required for definite purpose and not covered by these Specifications shall be in accordance to the relevant IRC or BIS or AASHTO/ASTM/BS Standards in order of priority. - Removal of Works Not Conforming to Specifications All materials or works not conforming to the requirements of the Specifications shall be considered unacceptable and rejected. The unacceptable materials or works that are rejected shall be immediately removed from the site and replaced with the specified quality materials/works, unless the defects are corrected and approved by the Engineer. If the Contractor fails to comply promptly with any order of the Engineer made under the provisions of this Clause, the Engineer has the authority to get it removed and replace the unacceptable materials or work at the cost of the Contractor. - Inspection of Materials at Source of Supply The Engineer may choose to inspect material at source of supply. In the event, the following conditions shall be met. - The Contractor and the manufacturer of material shall assist and co-operate with the Engineer in carrying out the inspection. - The Engineer shall have right to enter areas of plant where the manufacture or production of material is carried out as per IRC: SP:112. Procurement of materials (natural/factory manufactured)/finished products from a source by the Contractor shall be done after the source is approved by Engineer. While approving a source, Engineer shall ensure that the material meets the acceptance criteria with respect to all applicable quality characteristics of the material, there is availability of adequate quantity of materials and the manufactured materials/finished products meet the additional requirement of IRC: SP: 112. - Delivery, Storage and Handling of Materials - All materials shall be handled and stored in appropriate manner to preserve their quality and fitness for the work. During the handling of all aggregates or other construction materials, special care shall be taken to prevent contamination. Furthermore, aggregates shall be handled in such a manner as to prevent segregation. - Vehicles used in transporting construction materials shall be kept clean and in proper working condition so as to prevent the loss of materials during transportation and meet the requirements of the Specifications. - The Contractor shall not be allowed to store materials and equipment within the Right-of-Way (ROW) and or anywhere at road side. The Contractor shall have to acquire extra land for establishing the camp for the project which shall be used for storage purposes and for placing of construction materials, plant, equipment, work shop, testing laboratory and accommodation for workers, etc. Even for small Works such as maintenance, Contractor shall acquire a separate land for storage and placing of construction materials and equipment. - Materials Furnished by the Employer When the Contract provides that certain materials required to complete the Works will be supplied by the Employer, such material will be delivered or made available to the Contractor at the location(s) specified in the Contract. The Contractor shall be responsible for all damages occurring to the materials furnished by the Employer while the materials are in his possession. Any demurrage or storage charges shall also be the responsibility of the Contractor. The Contract shall include the cost of handling, transportation and placing of all Employer furnished materials in the Contract unit price for the relevant pay item of Bill of Quantities/Scope of Work. - Workmanship and Quality Control The Contractor shall be responsible to produce the Works that shall conform in quality and accuracy of detail to all the requirements of the Specifications and Drawings, and the Contractor shall, at his own expense, institute a quality control system and provide experienced engineers, foremen, surveyors, materials and laboratory technicians, other technicians and other technical staff, together with all transport, instruments and equipment to ensure adequate supervision and positive control of the Works at all times. The Contractor shall conduct tests or have them conducted continually on a regular basis, to check the properties of natural materials and processed natural materials and of products manufactured on the site, such as concrete and asphalt. Similarly, the quality of all elements of the Works shall be checked on a regular basis so as to ensure compliance with the specified requirements. The Contractor shall submit to the Engineer for examination, the results of all relevant tests, measurements and levels indicating compliance with the Specifications on completion of every part of the Work. Any lapse in maintaining the quality of Works shall be liable for penalty as per the provisions of Contract Agreement. - LAWS TO BE OBSERVED The Contractor shall observe and comply with all Central and State laws, local laws and ordinance which affect those employed on the work or affect the conduct of the work. The Contractor shall provide all safeguards, safety devices, and protective equipment and take any other actions necessary for safety and health of employees on the project. - PATENTED DEVICES, MATERIALS AND PROCESSES If the Contractor requires or desires with the approval of the Engineer to use any design, device, material or process covered by trademark, patent or copyright, the Contractor shall obtain the right for its use by legal agreement with the patentee or owner. A copy of the agreement shall be furnished to the Engineer. Contract prices shall include all royalties and costs arising from patents, trademarks and copyrights. - MATERIALS AND TEST STANDARDS The relevant standards for materials, as well as the testing procedures, have been indicated at appropriate places in the Specifications. A list of these standards with their full title are included at Appendix 2. - SIEVE DESIGNATIONS The sieve designations referred to in the Specifications correspond to those specified by Bureau of Indian Standards in IS:460. Specification of Wire Cloth Test Sieves shall be as per IS 460 Part 1 and Specification of Perforated Plate Test Sieves as per IS 460 Part 2. The details of the testing of sieves shall be as per IS:460 Part III. - CONSTRUCTION EQUIPMENT In addition to the conditions indicated in the Contract Agreement, the following conditions regarding use of equipment in Works shall be satisfied: - The Contractor shall be required to give a trial run of the equipment for establishing their capability to achieve the laid down Specifications and tolerances to the satisfaction of the Engineer before commencement of the work; - All equipment provided shall be of proven efficiency and shall be operated and maintained at all times in a manner acceptable to the Engineer; - Plants, equipment and instruments provided shall have adequate sensitivity, facility for calibration to desired level and shall be robust; - Plant, equipment and instrument provided shall have data logging arrangement and control systems to enable automatic feedback control of process; - Plants, equipment and instruments provided shall have adequate safety features and pollution control devices; - Plants, equipment and instruments provided shall be operated by skilled and qualified operators; - All the plants, equipment and instruments to be deployed on the Works shall be got approved from the Engineer for ensuring their fitness and efficiency before commencement of works; - Any plant/equipment/instrument not meeting the approval of the Engineer shall be removed from the site forthwith; - No plant/equipment/instrument shall be removed from site without permission of the Engineer; - The Contractor shall also make available stand by equipment and spare parts; - The Contractor shall also make arrangement of all required equipment at site for quality control work as detailed in IRC: SP:112 and as directed by the Engineer. - DRAWINGS - The drawings provided in the Contract Agreement shall be used only for specific purposes as mentioned in Contract Agreement. - The Contractor, based on his surveys and investigations, shall prepare and submit the working drawings (hard and soft copy) to the Engineer for each activity at least 45 days in advance of the scheduled date to the start of the activity as per his approved works programme. The working drawings shall clearly show the modifications, if any, proposed with reference to corresponding drawing in Contract Agreement (applicable for item rate contract only). - The Contractor shall submit documents and drawings, as listed below, of the various components/elements of the project highway and project facility: - Horizontal alignment and longitudinal profile. - Cross-section at required interval along the alignment, within ROW - Typical cross-sections with details of pavement structure. - Detailed working drawings for individual culverts. - Detailed working drawings for individual bridges, tunnels and structures. - Detailed drawings for improvement of at grade and grade-separated intersections, Interchanges, VUP, LVUP, PUP, ROB, flyovers - Drawing showing details of facilities for pedestrians including sidewalks, guard rails and crossing facilities - Drawings for road sign, markings showing the details of locations of signs with configurations and showing marking details at specific locations. - Drawings for toll plaza, street lighting, bus bays, parking areas, rest areas, O&M Centre, weighing stations etc. - Drawing of surface/ sub-surface drainage system - Drawings of traffic diversions plans and traffic control measures in work zone(s) - Drawings of slope protection measures, rain water harvesting - Drawings of landscaping and horticulture, if part of project - Other miscellaneous drawings like metal beam crash barrier, vehicular impact attenuator assembly, delineators and object markers etc. - Drawings or any other drawing required as per scope of Works. - All drawings required for implementation of Environmental Management Plan as specified. - As Built Drawings or any other scope of work drawings required - The Engineer shall approve/review the working design and drawings including the modifications proposed and ensure that they are in conformity or otherwise with the Scope of the Project and Specifications and Standards. The Engineer shall revise the design and drawings, if required, approve and issue to the Contractor two copies of Good for Construction (GFC) drawings/design at least 15 days in advance of the scheduled date of the start of the activity. No review and/or observation of Engineer and /or its failure to review and /or convey it observations on any drawings shall relieve the Contractor of its obligation and liability under the Contract Agreement in any manner. - The tendered rates/prices for the work shall be deemed to include the cost of preparation, supply and delivery of all necessary drawings, prints, soft copies which the Contractor is required to provide in accordance with the Contract. - SITE INFORMATION - The information about the site of work and site conditions provided in the Tender Documents is given in good faith for guidance only but it shall be the responsibility of the Contractor to satisfy himself regarding all aspects of site conditions. - The location of the Works and the general site particulars are as shown in the Site plan/Index plan enclosed with the Tender Documents. 110.3 Whereas the Right-of-Way to the bridge sites/road Works shall be provided to the Contractor by the Employer, the Contractor shall have to make his own arrangement for the land required by him for site offices, field laboratory, staff, site for plants and equipment maintenance and repair Workshop, construction workers’ camp, clinic, stores, generator for electricity, water supply arrangement, water storage tanks etc. - MOBILISATION AND DEMOBILIZATION 111.1 This work consists of performing all preparatory work and operations necessary to move personnel, equipment, supplies, and incidentals to the project site; to establish offices, buildings, and other facilities that are necessary for performing the work; and to accomplish all other work or operations that must be performed, including costs that must be incurred, to begin work on the project. moving personnel, equipment, material, and incidentals to the project and performing work necessary before beginning work at the project site. This work also includes obtaining permits, insurance, and bonds. - Performing all Works and operations necessary to accomplish final cleaning- up to move personnel, equipment, supplies, and incidentals from the project site; to remove all offices, buildings, and other facilities that were necessary for performing the Works; and to accomplish all other Works that must be performed, including costs that must be incurred, after acceptable completion of construction operations on the project - Final Cleaning/ Clearing of Work Sites: The Department will not consider the work complete and will not make final payment/ issue completion certificate until the Contractor clears the right-of-way, borrow pits, quarry and crusher sites, and all ground the Contractor occupies in connection with the work of all rubbish, equipment, excess materials, temporary structures, and weeds, place rubbish and all waste materials of whatever nature, other than hazardous materials, on either public or private property in a location out of view from the roadway and in a manner to the employer that does not present an unsightly appearance, restore in an acceptable manner all property, both public and private, that was damaged in the execution of the work, drain all ditches and all borrow pits where practical, and leave all space under structures unobstructed and in such condition that drift will not collect and induce scouring. - SETTING OUT 112.1 The work shall consist of surveying and staking for layout of work. The Contractor shall establish working bench marks (indicating x, y, z coordinates) which shall be tied with the nearest GTS benchmarks in the area soon after taking possession of the site. The Reference GTS benchmarks for the area shall be as indicated in the Contract Agreement and the values of the same shall be obtained by the Contractor from the Engineer. The working benchmarks shall be at the rate of four per km and also at or near all drainage structures, over-bridges and underpasses. The working benchmarks/levels should be got approved from the Engineer. Checks must be made on these benchmarks once every month and adjustments, if any, shall be got approved from the Engineer and recorded. An up-to- date record of all benchmarks including approved adjustments, if any, shall be maintained by the Contractor and also a copy supplied to the Engineer for his record. - The lines and levels of formation, side slopes, drainage Works, carriageways and shoulders shall be carefully set out and frequently checked, care being taken to ensure that correct gradients and cross-sections are obtained everywhere. - In order to facilitate the setting out of the Works, the centre line of the carriageway or highway must be accurately established by the Contractor and approved by the Engineer. It must then be accurately referenced in a manner satisfactory to the Engineer, at every 50 m intervals in plain and rolling terrains and 20 m intervals in hilly terrain and in all curve points as directed by the Engineer, with marker pegs and chainage boards set in or near the fence line, and a schedule of reference dimensions shall be prepared and supplied by the Contractor to the Engineer. These markers shall be maintained until the Works reach finished formation level and are accepted by the Engineer. - On construction reaching the formation level stage, the centre line shall again be set out by the Contractor and when approved by the Engineer, shall be accurately referenced in a manner satisfactory to the Engineer by marker pegs set at the outer limits of the formation. - No reference peg or marker shall be moved or withdrawn without the approval of the Engineer and no earthwork or structural work shall commence until the centre line has been referenced. - The Contractor will be the sole responsible party for safe-guarding all survey monuments, bench marks, beacons, etc. The Engineer will provide the Contractor with the data necessary for setting out the centre line. All dimensions and levels shown on the drawings or mentioned in documents forming part of or issued under the Contract shall be verified by the Contractor on the site and he shall immediately inform the Engineer of any apparent errors or discrepancies in such dimensions and levels. The Contractor shall, in connection with the staking out of the centre line, survey the terrain along the road and shall submit to the Engineer for his approval, a profile along the road centre line and cross-sections at intervals as required by the Engineer. The construction staking shall be done by personnel who are trained and experienced in construction layout and staking of the type and kind required in the Contract. Field notes shall be kept in standard, bound field notebooks as approved by the Engineer. Field notes shall be subject to inspection by the Engineer and shall be the property of the Employer. The Contractor shall correct any deficient staking or construction work which resulted from inaccuracies in the staking operations or from the Contractor’s failure to report inaccuracies in the plans or survey data furnished by the employer. - After obtaining approval of the Engineer, work on earthwork can commence. The profile and cross-sections as per Section 305, shall form the basis for measurements and payment. The Contractor shall be responsible for ensuring that all the basic traverse points are in place at the commencement of the contract and, if any, are missing, or appear to have been disturbed, the Contractor shall make arrangements to re-establish these points. A “Survey File” containing the necessary data will be made available for this purpose. If in the opinion of the Engineer, design modifications of the centre line or grade are advisable, the Engineer will issue detailed instructions to the Contractor and the Contractor shall perform the modifications in the field, as required, and modify the ground levels on the cross-sections accordingly as many times as required. For survey details IRC:52 and IRC: SP:19 shall be referred. There will be no separate payment for any survey work performed by the Contractor. The cost of these services shall be considered as being included in the rate of the items of work in the Bill of Quantities - Precision automatic levels, having a standard deviation of +2 mm per km, and fitted with micrometer attachment shall be used for all double run levelling work. Setting out of the road alignment and measurement of angles shall be done by using Total Station with traversing target, having an accuracy of one second. Measurement of distances shall be done (+/- 2 mm). using precision instruments like Distomat / Drone / LiDAR/Total Station/ GPS for accurate 3D data in electronic and printed format. - The work of setting out shall be deemed to be a part of general Works preparatory to the execution of work and no separate payment shall be made for the same. - PUBLIC UTILITIES AND GROUND CONDITIONS - Drawings scheduling the affected services like water pipes, sewers, oil pipelines, cables, gas ducts etc. owned by various authorities including Public Undertakings and Local Authorities included in the Contract Agreement shall be verified by the Contractor for the accuracy of the information prior to the commencement of any work. Investigation of ground condition deep in to the earth for locating utility lines in highway/road projects shall be carried out with the help of Ground Penetration Radar (GPR)/ MORTH Section 2400, IRC:SP:19 and for bridge projects with geophysical investigation as per IRC:123 and location of utilities may be checked and verified. The Contractor shall inform the Engineer that the utilities such as water supply, sewers, oil pipe lines, underground cables, gas ducts, electric poles which are to be relocated before commencing the work is carried out after the approval of the Engineer. The Contractor / Engineer shall notify all utility agencies who may have installation in the work area and secure their assistance in locating and identifying all utilities before starting any work that may cause any damage to such utilities with the approval of the Engineer. The Contractor shall pursue the Engineer for timely removal of utilities including overhead power transmission lines where necessary. The Contractor shall schedule work in such a manner as to protect existing utility facilities (functional/ nonfunctional) until they are relocated, abandoned or replaced. The Contractor shall ensure that all utilities encountered within the Right -of -Way i.e. OFC, telephone, power, water supply, sewerage or any others, remain operational at all times. Any utility, if damaged, due to construction operation, shall be promptly repaired by the Contractor at his cost. It shall be ensured that these utilities are laid underground at the extreme edge of R-O-W. - Notwithstanding the fact that the information on affected services may not be exhaustive, the final position of these services within the Works shall be supposed to have been indicated based on the information furnished by different bodies and to the extent the bodies are familiar with the final proposals. The intermediate stages of the Works are, however, unknown at the design stage, these being dictated by the Contractor’s methods of working. Accordingly, the Contractor’s programme must take into account the period of notice and duration of diversionary Works of each body/organization as given on the Drawings and the Contractor must also allow for any effect of these services and alterations upon the Works and for arranging regular meetings with the various bodies at the commencement of the Contract and throughout the period of the Works, the Contractor shall have no objection if the public utility bodies vary their decisions in the execution of their proposals in terms of programme and construction, provided that, in the opinion of the Engineer, the Contractor has received reasonable notice thereof before the relevant alterations are carried out. - No removal of or alterations to the utility shall be carried out unless written instructions are issued by the Engineer. - Any services affected by the Works must be temporarily supported by the Contractor who must also take all measures reasonably required by the various bodies to protect their services and property during the progress of the Works. - The Contractor may be required to carry out certain Works for and on behalf of various bodies, which he shall provide, with the prior approval of the Engineer. - The work of temporarily supporting and protecting the public utility services during execution of the Works shall be deemed to be part of the Contract and no extra payment shall be made for the same. - The Contractor shall be responsible to co-ordinate with the service providers for cutting of trees, shifting of utilities, removal of encroachments etc. to make site unencumbered for completion of work. This will include frequent follow-up meetings. Co- ordination for making project site unencumbered shall be deemed to be part of the Contract and no extra payment shall be made for the same. - In some cases, the Contractor may be required to carry out the removal or shifting of certain services/utilities on specific orders from the Engineer for which payment shall be made to him. Such Works, however, shall be taken up by the Contractor only after obtaining clearance from the Engineer and ensuring adequate safety measures. - ENVIRONMENTAL PROTECTION 114.1 General The Contractor shall take all precautions for safeguarding the environment during the course of the construction of the works. He shall abide by all laws, rules and regulations in force governing pollution and environmental protection that are applicable in the area where the works are situated. - Borrow Pits for Embankment Construction Borrow pits shall be selected only after testing the suitability of materials for use in construction and shall not be dug in the Right-of-Way of the road. The stipulations in Section 305.2.2 shall govern. The borrow pits shall not be left in a condition likely to cause hazard to human and animal life. The Contractor shall seek prior approval from the concerned authorities for operating the borrow pits. - Quarry Operations The Contractor shall obtain materials from quarries only after obtaining the consent of the Mining Department or other concerned authorities of the State Government. The quarry operations shall be undertaken within the purview of the rules and regulations in force. - Control of Soil Erosion, Sedimentation and Water Pollution The Contractor shall carry out the works in such a manner that soil erosion is fully controlled, and sedimentation and pollution of natural water courses, ponds, tanks and reservoirs is avoided. The stipulations in Clause 306 shall govern. - 114.5 Pollution from Plants and Batching Plants Stone crushing and screening plants, bituminous hot-mix plants, concrete batching plants etc. shall be located sufficiently away from habitation, agricultural operations fields or industrial establishments. The locations shall be as permissible under the laws governed by local bodies/ administration of the area. The Contractor shall take every precaution to reduce the levels of noise, vibration, dust and emissions from his plants and shall be fully responsible for any claims or damages caused to the owners of property, fields and residences in the vicinity and violation of pollution control norms, if any. - Substances Hazardous to Health The Contractor shall not use or generate any materials in the works which are hazardous to the health of persons, animals or vegetation. Where it is necessary to use some substances which can cause injury to the health of workers, the Contractor shall provide protective clothing or appliances to his workers. - Use of Density Gauges Density gauges shall be used only where permitted by the Engineer after calibration as per IRC:SP:112. The Contractor shall provide the Engineer with a copy of the regulations governing the safe use of density gauges he intends to employ and shall abide by such regulations. - Environmental Protection - The Contractor shall take all steps to minimize dust nuisance during the construction of the works along the haul roads and the Worksites by sprinkling water at a frequency specified by the Engineer, as shown in Annexure II. All existing highways and roads used by vehicles or equipment of the Contractor or any of his sub-Contractors or suppliers of materials or plant, and similarly any new roads which are part of the Works and which are being used by traffic, shall be kept clean and clear of all dust/mud or other extraneous materials dropped by the said vehicles. Similarly, all dust/mud or other extraneous materials from the works spreading on these highways shall be immediately cleared by the Contractor Clearance shall be done immediately by sweeping and removal of all dust, mud and other debris entirely from the road surface including shoulder, median island, drainage spouts on decks or other drainage systems. Additionally, if so, directed by the Engineer, the road surface shall be hosed or watered using suitable equipment. Damages to Existing Roads: Any structural or functional damages and loss of riding surface caused to the existing roads by the Contractor’s construction vehicles/ equipment shall be made good without any extra cost to the employer. Compliance with the foregoing will not relieve the Contractor of any responsibility for complying with the requirements of any authority in respect of the roads used by him. - Air Quality The Contractor shall device, implements methods of working to minimize dust, gaseous and other air-borne emissions and carry out the Works in such a manner as to minimize adverse impacts on the air quality. The Contractor shall use effective water sprays during delivery, manufacture, processing and handling of materials when dust is likely to be created, and to dampen stored materials during dry and windy weather. Stockpiles of friable materials shall be covered with clean tarpaulins, with applications of sprayed water during dry and windy weather. Stockpiles of materials or debris shall be dampened prior to their movement, except where this is contrary to the Specification. Any vehicle with open load-carrying area used for transporting potentially dust-producing material shall have properly fitting side and tail boards. Materials having the potential to produce dust shall not be loaded to a level higher than the side and tail boards and shall be covered with clean tarpaulins in good condition. The tarpaulin shall be properly secured and extend at least 300 mm over the edges of the side and tail boards. - Water Sources and Water Quality The Contractor shall provide independent sources of water supply, such as bore wells, for use in the Works and for associated storage, Workshop and work force compounds. Prior approval shall be obtained from the relevant State Authorities and all installations shall be in compliance with local regulations. The Contractor shall protect all watercourses, waterways, ditches, canals, drains, lakes, reservoirs and the like from pollution as a result of the execution of the Works. All water and other liquid waste products like petroleum products and chemicals arising on the Site shall be collected and disposed of at a location on or off the Site and in a manner that shall not cause either nuisance or pollution. The Contractor shall at all times ensure that all existing stream courses and drains within and adjacent to the Site are kept safe and free from any debris and any materials arising from the Works. The Contractor shall not discharge or deposit any matter arising from the execution of the Works into any water course except with the permission of the Engineer and the regulatory authority concerned. - Construction Camps The construction camps shall conform to the State and National building regulations as applicable. The area for the storage of polluted materials shall be stored on impervious floors and shall be surrounded by impervious ditches in order to avoid spilling of polluted material to surrounding areas. Construction camps shall be properly arranged to avoid noise pollution to the nearby habitants and to avoid contamination of water courses from wastewater drainage. To prevent such contamination, waste water generated at the campsites shall be discharged into soak pits. Human excreta shall be treated though septic tanks prior to discharge and shall conform to directives and guidelines of the State Water accumulated in tyres, empty vessels and containers of all nature will be regularly cleaned to avoid the related health hazards. The Contractor shall provide and maintain in a neat and sanitary condition accommodations for the use of the employees and workers as may be necessary to comply with the requirements of Central, State, and local regulations. Spilling of oil and bituminous products during construction and transport shall be avoided to reduce the chances of contamination of surface as well as ground water. - Occupational Health & Safety of the Workforce The Contractor shall prepare and submit to the Engineer the Occupational Health & Safety Procedures / Practices for the workforce in all quarry sites, plant sites, work sites, camp sites, etc., in accordance with the applicable laws. - Control and Disposal of Wastes The Contractor shall control the disposal of all forms of waste generated by the construction operations and in all associated activities. No uncontrolled deposition or dumping shall be permitted. Wastes to be so controlled shall include, but shall not be limited to, all forms of fuels and engine oils, all types of bitumen, cement, surplus aggregates, gravels, bituminous mixtures etc. The Contractor shall make specific provision for the proper disposal of these and any other waste products, conforming to local regulations and acceptable to the Engineer. - Transport of Hazardous Materials Transport of hazardous materials, in bulk or in sealed containers, shall meet the requirements of the State regulations. Prior to ordering transport of hazardous material in bulk, the Contractor must obtain the approval of the relevant authority as well as of the Engineer. The transport of diesel, petrol, gaseous material, chemical and explosives for quarrying shall be governed by safety laws of the local authorities. Precautionary measures and conformity with regulations shall be stated in a Method Statement for the approval of the Engineer. Sealed containers of hazardous materials shall be stored in a well-ventilated room, well guarded and secured. - Emergency Response The Contractor shall plan and provide remedial measures to be implemented in the event of occurrence of emergencies such as spillages of oil or bitumen or chemicals, fire. The Contractor shall provide the Engineer with a statement of the measures he intends to implement in the event of such an emergency, which shall include a statement of how he intends to provide personnel adequately trained to implement such measures. - Measurement for Payment The compliance of all provisions made in the Clause 114 shall be deemed to be incidental to the work and no separate measurement or payment shall be made. The Contractor shall be deemed to have made allowance for all such compliance with these provisions in the preparation of his bid for items of work included in the Bill of Quantities and full compensation for such compliance shall be deemed to be covered by the Contract price. - ARRANGEMENT FOR TRAFFIC DURING CONSTRUCTION - General Contractor shall perform the works in a manner that ensures the safety and convenience of the public and protects the residents and property adjacent to the project accommodate public traffic on roads adjacent to and within the project until the project is completed and accepted. The Contractor shall at all times, carry out work on the highway in a manner creating least interference to the flow of traffic while consistent with the satisfactory execution of the same. For all works involving improvements to the existing highway, construction/ reconstruction of culverts, at-grade, grade separated structure, the Contractor shall, as specified and in accordance with the directives of the Engineer, provide and maintain, during execution of the work, a passage for traffic either along a part of the existing carriageway under improvement or along a temporary diversion constructed close to the highway. Before taking up any construction (especially of brownfield project) or maintenance operation, the Contractor shall prepare a Work Zone Traffic Management Plan (WTMP) as recommended by IRC SP 55, for each work zone and submit it to the Engineer for prior approval. WTMPs shall be prepared by persons knowledgeable and experienced with respect to the fundamental principles of road safety, and the road work activities to be performed. This plan should include inter alia: - Considerations and provision for highest regard to traffic safety as well as to provide a safe working environment to the workmen. - Provision of a qualified safety officer with support staff to serve as a site safety team - Clearly defining the roles and legal responsibilities of all the involved stakeholders - Training of workers on their safety and safety of moving traffic - Ensure timely passage of an emergency vehicle through the work zone - If the Contractor proposes a road closure, he shall provide an alternative routing of the traffic which must be approved by the Engineer. - Guidance and protection for Vulnerable Road Users - Provision of traffic control and safety devices in construction zones as per IRC: SP: 55 and other applicable IRC Codes - Safety measures for the workers engaged including personal protection equipment - First aid and emergency response arrangements - Details and drawings of arrangements in compliance with other sub-Sections of this Section. - Traffic Control and Safety Devices for Work Zones Following Traffic Control devices shall be used for efficient and safe movement of traffic through the Work Zone. Their application (as per the site specific requirements) and standards shall be as given in IRC:SP:55: - Traffic Signs: These shall be conforming to IRC:67 and to those additional ones prescribed by IRC:SP:55, specifically for Work Zones. - Traffic Cones - Tubular Markers - Hazard markers/ sharp deviation signs - Drums (Empty bitumen drums shall never be used under any circumstances) - Water filled Barricades - New Jersey Barriers - Barricades. (Type IV barricades comprising MS sheets and channel shall not be used for segregation since they provide no protection). - Direction indicator barricades - Delineators - Temporary pavement markings and road studs - Floodlights - Flashing warning beacons - Temporary traffic control signals - Portable variable message signs - Arrow boards (flashing and or sequential displays. - Speed reduction measures 115.3 The common construction activities on existing highways/roads would have following situations of work zones: - Widening existing single lane road to two lane road. - Widening two lane road to four lane road - Widening four lane road to six lane road - Diversion especially at cross drainage structures - Maintenance of roads, in the middle or at edges - Detours 115.4 IRC:SP:55 gives details of layout for each of above situations with sequence of construction, traffic signs (placement and configurations), barricades, flags, lights, markings. Traffic control devices shall be maintained during the period of construction. The portion/ stretch of the road and any diversion to be used by the traffic moving/ negotiating through the work zone shall be of pavement structure as the road stretches on either side. - Passage of Traffic along a Part of the Existing Carriageway under improvement For widening/strengthening existing carriageway where part width of the existing carriageway is proposed to be used for passage of traffic unpaved shoulders shall be provided on the side on which work is not in progress. The treatment to the shoulder shall consist of providing at least 150 mm thick granular (Wet Mix Macadam) base course covered with bituminous premix carpet with seal coat / surface dressing, in a width of at least 1.5 m and the unpaved shoulder shall be maintained throughout the period during which traffic uses the same to the satisfaction of the Engineer. The continuous length, in which such work shall be carried out, would be limited normally to 500 m at a place. However, where work is allowed by the Engineer in longer stretches passing places at least 20 m long with additional paved width of 2.5 m shall be provided at every 0.5 km interval. In case of eccentric widening of existing two-lane to four-lane, the existing two-lane road shall be strengthened to carry the diverted traffic plus construction traffic plus normal traffic on the carriageway. Thereafter the work on construction of additional two-lane shall be taken up. In case of concentric widening, stipulations as in paragraph above shall apply. After the Works are completed, with the approval of the Engineer, the unpaved shoulder shall be dismantled, the debris disposed of and the area cleared as per the direction of the Engineer. - Passage of Traffic along a Temporary Diversion In stretches where it is not possible to pass the traffic on part width of the carriageway, a temporary diversion shall be constructed with 7 m carriageway and 2.5 m unpaved shoulders on each side (total width of roadway 12 m) with the following provision for road crust in the 7 m width: - Earthwork - 200 mm (compacted) granular sub-base - 225 mm (compacted) granular base course - Priming and Tack Coat and - 30 mm bituminous concrete The location of such stretch, alignment and longitudinal section of diversion including junctions and temporary cross drainage provision shall be as approved by the Engineer. 115.7 Traffic Safety and Control The Contractor shall take all necessary measures for the safety of traffic during construction and provide, erect and maintain such barricades, including signs, marking, flags, lights and flagmen as per the work zone traffic management plan WTMP submitted by the Contractor and approved by the Engineer, referred to in Sub-Section 115. Before taking up any construction, an agreed phased programme for the diversion of traffic on the highway shall be as per the traffic management plan All construction equipment working or parked on or within the traffic lanes or shoulders under running traffic conditions shall be equipped with flashing yellow beacons. The Contractor shall conduct all operations to minimize any drop-offs (abrupt changes in roadway) exposed to traffic. Drop-offs in the travelled way shall be protected by a wedge of compacted stable material capable of carrying traffic (the wedge being 1 vertical to 4 horizontal or flatter). The Engineer shall authorize other methods, to protect drop-offs when conditions do not allow a wedge of compacted, stable material. Warning signs, barricades, warning lights, and all other traffic control devices shall not be removed if the hazard has not been eliminated. Only upon receipt of specific written authorization from the Engineer, the Contractor may remove or cease to maintain warning signs, barricades, warning lights, and all other traffic control devices. The barricades erected on either side of the carriageway/portion of the carriageway closed to traffic, shall be of strong design to resist violation, and painted with alternate black and white stripes. Red lanterns or warning lights of similar type shall be mounted on the barricades at night and kept lit throughout from sunset to sunrise. At the points where traffic is to deviate from its normal path (whether on temporary diversion or part width of the carriageway) the channel for traffic shall be clearly marked with the aid of pavement markings, direction signs boards as per IRC:67or a similar device as per the work zone traffic management plan / to the directions of the Engineer. At night, the passage shall be delineated with lanterns or other suitable light source including solar energy bulbs. One-way traffic operation shall be established whenever the traffic is to be passed over part of the carriageway where it is inadequate for two-lane traffic. This shall be done with the help of temporary traffic signals or flagmen kept positioned on opposite sides during all hours. For regulation of traffic, the flagmen shall be equipped with red and green flags and lanterns/lights. On both sides, suitable regulatory/warning signs as approved by the Engineer shall be installed for the guidance of road users. On each approach, at least two signs shall be put up, one close to the point where transition of carriageway begins and the other 120 m away. The signs shall be of approved design and of reflective type, as directed by the Engineer 115.8 Widening Existing Single/ intermediate Lane Road to Two Lane Road. This strategy involves treating the shoulder on one side while utilizing part of the existing carriageway for traffic movement. In the first phase, at least 1.5 m of shoulder width shall be treated with a pavement thickness equal to that of the main carriageway. It shall be ensured that a minimum trafficable width of 5.5 m by combining the treated shoulder and part of the existing carriageway is available for allowing for smooth traffic flow during construction. In the second phase, traffic will be diverted onto the improved width, and the remaining portion of the carriageway will be upgraded. Traffic through the work zone shall be managed using proper placement of traffic signs, water-filled barricades, New Jersey barriers, and temporary markings as specified in IRC: SP: 55. Upon completion of the work and approval by the Engineer, the treated unpaved shoulder shall be dismantled if it obstructs the free flow of traffic. The debris shall be disposed of, and the area cleared as per the Engineer’s instructions. All design and construction activities shall comply with the relevant IRC codes. 115.9 Widening Two Lane Road to Four Lane Road The widening of the existing two lane road to the four-lane road can be done in two ways depending upon the site requirement, In both the situations, the work shall be carried out in two stages. - For eccentric widening, the new two-lane carriageway (as per the design) shall be constructed on planned one side of existing carriageway with traffic moving on the existing carriageway. In the second phase, the traffic shall be allowed on the newly constructed carriageway with median cross over and existing carriageway shall be strengthened to the designed strength. The traffic shall be guided through the work zone in both the stages, by correct placement of traffic signs, water filled barricades, new jersey barrier, temporary markings as shown in IRC: SP:55. - For Widening Two Lane Road to Four Lane Road for concentric widening, the width of service roads shall be at least 3.5 m or more if required from traffic considerations"with pavement thickness designed for the expected traffic (accounting for directional distribution). In the second stage, the new four lane designed highway shall be built in the island between the two service roads while traffic plying unidirectional on service roads. The traffic shall be guided through the work zone in both the stages, by correct placement of traffic signs, water filled barricades, new jersey barrier, temporary markings as shown in IRC:SP:55. The Works shall advance in the same sequence on the next stretch. The design and construction shall be as specified in relevant IRC codes - Widening Four Lane Road to Six Lane Road For Widening Four Lane Road to Six Lane Road for concentric widening, the width of service roads shall be at least 3.5 m or more if required from traffic considerations, pavement thickness designed for the expected traffic. In the second stage, the additional one lane for each carriageway along with the median shall be taken up. The transition from existing carriageway to the service road would be carefully planned for smooth traffic flow. The traffic shall be guided through the work zone in both the stages, by correct placement traffic signs, water filled barricades, new jersey barrier, temporary markings as shown in IRC: SP:55. The design and construction shall be as specified in relevant IRC codes - Diversion Especially at Cross Drainage Structures or Utilities The length of the diversion shall be of the shortest practical length having regard to gradient and obstruction and shall be sited as agreed between the Engineer and the Contractor. The width of carriageway shall be 7 m carriageway and 2.5 m unpaved shoulders on each side (total width of roadway12 m) with pavement thickness to withstand the expected traffic. The gradient of any deviation shall not be greater than10% except with the express approval of the Engineer, and any acute intersection of gradient shall be properly graded to a smooth vertical curve, to the satisfaction of the Engineer. The traffic shall be guided through the work zone in both the stages, by correct placement traffic signs, plastic cone, hazard markers, barricades, temporary markings as shown in IRC:SP:55. The design and construction shall be as specified in relevant IRC codes - Maintenance of Diversions and Traffic Control Devices Signs, lights, barriers and other traffic control devices, adequate lighting and other arrangements, as well as the riding surface of diversions and unpaved shoulders shall be maintained in a satisfactory condition till such time they are required and as directed by the Engineer. The temporary travelled way shall be kept free of dust by frequent applications of water, if necessary. - Measurements for Payment and Rate All arrangements, as contained in this section (115) for safety of road users, during construction including provision of temporary diversions/temporary cross drainage structures/unpaved shoulders shall be measured and paid as per the BOQ. However, their maintenance, dismantling and clearing debris shall be considered as incidental to the Works and shall not be paid separately. - GENERAL RULES FOR THE MEASUREMENT OF WORKS FOR PAYMENT - General All measurements shall be made in the metric system. Different items of work shall be measured in accordance with the procedures set forth in the relevant Sections read in conjunction with the General Conditions of Contract. The same shall not, however, apply in the case of lumpsum contracts (Engineering, Procurement & Construction). All measurements and computations, unless otherwise indicated, shall be carried nearest to the following limits: - length and width 10 mm - height, depth or thickness of: - earthwork, subgrade, 5 mm - sub-bases, bases, surfacing, 5 mm - structural members 2.5 mm - areas 0.01 sqm - volume 0.01 cum In recording dimensions of work, the sequence of length, width and height or depth or thickness shall be followed. - Measurement of Lead for Materials Where lead is specified in the Contract for construction materials, the same shall be measured as described hereunder: Lead shall be measured over the shortest practicable route and not the one actually taken and the decision of the Engineer in this regard shall be taken as final. Distances upto and including 100 m shall be measured in units of 50 m, exceeding 100 m but not exceeding 1 km in units of 100 m and exceeding 1 km in units of 500 m, the half and greater than half of the unit shall be reckoned as one and less than half of the unit ignored. In this regard, the source of the material shall be divided into suitable blocks and for each block, the distance from the centre of placing pertaining to that block shall be taken as the lead distance. - Measurement of Pavement Thickness for Payment on Volume Basis The finished thickness of sub-bases, base and bituminous layers and concrete courses to be paid on volume basis shall be computed in the following manner: Levels shall be taken before and after construction, at the grid of points 10 m centre-to-centre longitudinally in straight reaches and 5 m centre-to-centre at curves. Normally, on two-lane roads, the levels shall be taken at four positions transversely, at 0.75 m and 2.75 m from either edge of the carriageway and on single-lane roads, these shall be taken at two positions transversely, being at 1.25 m from either edge of the carriageway. For multi-lane roads, levels shall be taken at two positions transversely for each lane. The transverse position for levels shall be 0.75 m from either edge of the carriageway and the remaining locations shall be at equi-distance in the balance portion of carriageway. For paved shoulder an additional level shall be taken at the centre of the shoulder. Suitable references for the transverse grid lines should be left in the form of embedded bricks on both ends or by other means so that it is possible to locate the grid points for level measurements after each successive course is laid. For pavement courses laid only over widening portions, at least one line of levels shall be taken on each strip of widening, or more depending on the width of widening as decided by the Engineer. Notwithstanding the above, the measurements may be taken at closer intervals also, if so desired by the Engineer, the need for which may arise particularly in the case of estimation of the volume of the material for profile corrective course (leveling course). The average thickness of the pavement course in any area shall be the arithmetic mean of the difference of levels before and after construction at all the grid points falling in that area, provided that the thickness of finished work shall be limited to those shown on the drawings or approved by the Engineer in writing. As supplement to level measurements, the Engineer shall have the option to take cores/make holes to check the depth of construction. The holes made and the portions cut for taking cores shall be made good by the Contractor by laying fresh bituminous mix/material of same quality in case of flexible pavement /concrete mix of same quality in case of rigid pavement and white topping/ or with other material as specified by the Engineer including compacting as required at his-own cost immediately after the measurements are recorded. - Checking of Pavement Thickness for Payment on Area / Volume Basis Where payment for any bituminous course in Section 500 is allowed to be made on the area basis, the Engineer may have its thickness checked with the help of a suitable penetration gauge/core cutting at regular intervals or other means as directed by the Engineer. - Measurement of Bituminous Courses for Payment on Weight Basis Plant-mixed bituminous materials for pavement courses, where specifically designated in the contract to be paid on weight basis, shall be weighed on accurate scales approved by the Engineer. Approved scales shall mean scales that are of size, capacity, kind and type suitable for the weighing to be done, and these shall be properly installed and maintained. Prior to the use of the scales and as frequently thereafter as the Engineer may deem necessary to ensure accuracy, the scales shall be checked and approved by the Engineer, or the Engineer may direct the Contractor to have the scales checked/calibrated by other competent agency at the cost of the Contractor. Location of the scales shall be as designated by the Engineer. Trucks used for hauling the material to be weighed shall be weighed empty daily at such times as the Engineer directs, and each truck shall bear a plainly legible identification mark For materials specified to be measured by weight, the Engineer will have the option to make measurements of the finished work by volume in accordance with Section 113.3 and such volumes shall be converted into weight for payment purposes. The factor for conversion from volume measurement to weight measurement shall be computed from the representative density of the compacted material at site determined at locations approved by the Engineer. - SCOPE OF RATES FOR DIFFERENT ITEMS OF WORK - For item rate contracts, the contract unit rates for different items of work shall be payment in full for completing the work to the requirements of the Specifications including full compensation for all the operations detailed in the relevant Sections of these Specifications under “Rates”. In the absence of any directions to the contrary, the rates are to be considered as the full inclusive rate for completed work covering all labour, materials, wastage, temporary work, plant, equipment, over-head charges and profit as well as the general liabilities, performance of other obligations, insurance and risks arising out of the Conditions of Contract. - The item rates quoted by the Contractor shall, unless otherwise specified, also include compliance with/supply of the following: - General Works such as setting out, clearance of site before setting out and clearance of Works after completion; - A detailed programme using modern project management software for the construction and completion of the work giving, in addition to construction activities, detailed network activities for the submission and approval of materials, procurement of critical materials and equipment, fabrication of special products/equipment and their installation and testing, for all activities of the Engineer/Employer that are likely to affect the progress of work, etc., including updating of all such activities on the basis of the decisions taken at the periodic site review meetings or as directed by the Engineer; - Samples of various materials proposed to be used on the Works for conducting tests thereon as required as per the provisions of the contract and MORTH Section 900 and IRC:SP:112. - Design of mixes as per the relevant Sections of the Specifications giving proportions of ingredients, sources of aggregates and binder along with accompanying trial mixes as per the relevant Sections of these Specifications to be submitted to the Engineer for his approval before use on the Works; - Cost of laying trial stretches; - Detailed drawings as per Clause 107; - Detailed design calculations and drawings for all Temporary Works (such as form-work, staging, centering, specialized constructional handling and launching equipment and the like); - Detailed drawings for templates, support and end anchorage, details for pre- stressing cable profiles, bar bending and cutting schedules for reinforcement, material lists for fabrication of structural steel, etc; - Mill test reports for all mild and high tensile steel and cast steel as per the relevant provisions of the Specifications; - Testing of various materials, mixes, and finished items as required under these Specifications and furnishing test reports/certificates as per IRC: SP:112 or as specified/as approved by the Engineer. - Inspection Reports in respect of formwork, staging, reinforcement and other items of work as per the relevant Specifications; - Any other data which may be required as per these Specifications or the Conditions of Contract or any other annexures/schedules forming part of the Contract; - Any other item incidental to work which is necessary for complying with the provisions of the Contract; - All temporary Works, formwork and false work not included as separate item in the BOQ; - Establishing and running a laboratory with facilities for testing for various items or Works as specified in Section 900 and other relevant Sections; - Cost of in-built provisions for Quality Assurance; - Cost of safeguarding the environment; and - Cost of providing “as-built drawings” in original and two sets of prints along with soft copy. - Portions of road Works beyond the limits and/or any other work may be got constructed by the Employer directly through other agencies. Accordingly, other agencies employed by the Employer may be working in the vicinity of the Works being executed by the Contractor. The Contractor shall liaise with such agencies and adjust his construction programme for the completion of work accordingly and no claim or compensation due to any reason whatsoever will be entertained on this account. The Employer will be indemnified by the Contractor for any claims from other agencies on this account. - METHODOLOGY AND SEQUENCE OF WORK - Prior to start of the construction activities at site, the Contractor shall, within 28 days after the date of the agreement unless otherwise stipulated in the Contract, submit to the Engineer for approval, the detailed method statement. The method statement shall be submitted in two parts. - The general part of the method statement shall describe the Contractor’s proposals regarding preliminary Works, common facilities and other items that require consideration at the early stage of the contract. The general part shall include information on: - Terrain, topography and climatic condition of project area; - Sources of materials like soil, coarse aggregates and fine aggregates, quantity and quality of materials available in different sources; - Sources of manufactured materials like bitumen, cement, steel reinforcement, pre-stressing strands and bearings etc. as per IRC:SP:112. The Contractor shall also submit the samples/test certificates of materials for consideration of the Engineer; - Locations of the site and other facilities such as camp site, sites, source of water, batching plant, hot mix plant, crushing plant, etc.; - Details of facilities available for transportation of men/material and equipment; - Information on procedure to be adopted by the Contractor for prevention and mitigation of negative environmental impact due to construction activities; - Work Zone Traffic Management Plan (WTMP) for Safety and traffic arrangement during construction as per IRCSP:55: - Implementation of activities provided in the Environmental Management Plan; - Any other information required by the Engineer. 118.3 The general part of the QA programme under Section 105.3 shall accompany the method statement. Special part of the method statement along with RFI (Report for Information) format as per IRC:SP: 112 shall be submitted to the Engineer by the Contractor for each important item of work as directed by the Engineer. The statement shall be submitted at least 4 weeks in advance of the commencement of the activity of item of work unless otherwise stipulated in the contract. The statement shall give information on: - Details of the personnel both for execution and quality control of the work; - Equipment deployment with details of the number of units, capacity, standby arrangement; - Sequence of construction and details of temporary or enabling Works like diversion of traffic, cofferdam, formwork including specialized formwork for superstructure, details of borrow areas, method of construction of embankment, sub-grade and pavement, pile concreting, proprietary processes and products and equipments to be deployed. Wherever required technical literature, design calculations and drawings shall be included in the method statement; - Testing and acceptance procedure including documentation; - The special part of the QA programme under sub-Section 105.3 for the particular item of work shall accompany the method statement for the concerned activity. The Engineer shall examine and approve the method statement with the required modifications. The modified method statement if required shall be submitted within 14 days of the receipt of the Engineer’s approval. The sole responsibility for adequacy and safety of the method adopted by the Contractor shall rest on the Contractor irrespective of any approval given by the Engineer. 118.4 Approval of Proprietary Products/ Processes/ Systems Within 90 days of the signing of agreement, the Contractor shall submit the following information for all proprietary products, process or any other item proposed to be used in the work, for approval of the Engineer. - Name of the manufacturer and name of the product/ process/system along with authenticated copies of the license/collaboration agreement; - General features of the product/process/system; - Details of the product development and development testing; - Acceptance test and criteria; - Installation procedure; - Maintenance procedure and schedule; - Warranty proposal. The Engineer may order additional test for the purpose of acceptance. Additional charges for test, if any, for the product/process/system shall be borne by the Contractor. For manufactured materials, used in road construction, tests to be conducted shall be as per applicable codes of practice / Annexure 5.1 of IRC: SP:112. Tentative list of factory manufactured finished items used in road construction and operation are indicated in Annexure 5.2 of IRC: SP:112, along with testing requirement for the same. The Contractor shall take up appropriate license from the concerned Authority for the transportation, storage and blasting. 119 CRUSHED STONE AGGREGATES Where the terms crushed gravel/shingle, crushed stone, broken stone or stone aggregate appear in any part of the Contract Documents or Drawings issued for work, they refer to crushed gravel/crushed shingle/crushed stone aggregate obtained from integrated crushing plant having appropriate primary crusher, secondary cone crusher, vertical shaft impactor capable of producing cubical aggregates and vibratory screen unless specified otherwise. Stone retained on 4.75 mm sieve shall have at least two faces fractured. - SUPPLY OF QUARRY SAMPLES Raw and processed samples of the mineral aggregates from the approved quarry shall be submitted by the Contractor at his cost. - APPROVAL OF MATERIALS Approval of all sources of material for work shall be obtained in writing from the Engineer before their use on the Works. - USE OF PAVEMENT SURFACES BY TRAFFIC - Ordinarily, no construction traffic shall be allowed on pavement under construction unless authorized by the Engineer. Even in that case, the load and intensity of construction traffic should be so regulated that no damage is caused to the sub-grade or pavement layers already constructed. Where necessary, service roads shall be constructed for this purpose and the same shall be considered as incidental to the work. 122.2 The wheels or the tracks of plant moving over the various pavement courses shall be kept free of deleterious materials. 122.3 Bituminous base course shall be kept clean and uncontaminated as long as the same remains uncovered by a wearing course or surface treatment. The only traffic permitted access to the base/binder course shall be that engaged in laying and compacting the wearing course or that engaged on such surface treatment where the base/binder course is to be blinded and/or surface dressed. Should the base/binder course or tack coat on the base/binder course become contaminated, the Contractor shall make good by cleaning it to the satisfaction of the Engineer, and if this is impracticable, by removing the layer and replacing it to Specifications without any extra cost to the Employer. - On Dry Lean Concrete sub-base, no heavy commercial vehicles like trucks and buses shall be permitted after its construction. No vehicles shall be allowed if polythene sheet is spread over DLC or over DLC also if construction of bonded rigid pavement with DLC as per IRC 15/IRC:58. Also, no vehicular traffic shall be allowed on a finished concrete pavement before the design period or of minimum 28 days of its construction and until the joints are cut and permanently sealed and cured. Even after 28 days of construction approval of the Engineer shall be taken for plying of the traffic. - FIELD LABORATORY - Scope The work covers the provision and maintenance of an adequately equipped field laboratory as required for site control on the quality of materials and the Works. - Description The Contractor shall arrange to provide fully furnished and adequately equipped field laboratory. The field laboratory shall preferably be located adjacent to the site office of the Engineer and provided with amenities like water supply, electric supply etc. as for the site office of the Engineer as described in this Section. The layout and size of the field laboratory shall be as indicated in the drawings or as per IRC:SP:112. In case no drawings is furnished, the laboratory shall include space for the storage of samples, equipment, laboratory tables and cupboards, working space for carrying out various laboratory tests, a wash basin, toilet facility and a curing tank for the curing of samples, around 4 m x 2 m x 1 m in size and a fume chamber. Wooden/concrete working table with a working platform area of about 1 m x 10 m shall be provided against the walls. Wooden cupboards above and below the working tables shall be provided to store accessories such as, sample moulds etc. At least 4 racks of slotted angles and M.S. sheets the size 1800 mm x 900 mm x 375 mm and at least 6 stools for laboratory test operators shall also be provided. The items of laboratory equipment shall be provided in the field laboratory depending upon the items to be executed as per Table 100-2. - Ownership The field laboratory building and equipment shall be the property of the Contractor. The Employer and the Engineer shall have free access to the laboratory. 123.4 Maintenance The Contractor shall arrange to maintain the field laboratory in a satisfactory manner until the issue of Taking Over Certificate for the completed work. Maintenance includes all activities described in Section 120.4. Table 100- 2 List of Laboratory Equipment Sl. No; Name of the Item; Number. A; GENERAL. 1; Weigh Balances. a); 5 – 20 kg capacity Electronic type – Accuracy 1 gm.; 1 No.. b); 500 gm capacity–Electronic Type Accuracy 0.01 gm; 1 No.. c); Electronic 5 kg capacity Accuracy 0.5 gm; 1 No.. d); Platform Balance scale-300 kg capacity accuracy 30 gm; 1 No.. e); Chemical Balance 100 gm capacity-accuracy 0.001 gm; 1 No.. 2; Oven-electrically operated, thermostatically controlled (including thermometer), stainless steel interior from 0°C to 220°C Sensitivity 1°C; 1 No. 3; Sieves : as per IS:460. a); I.S. sieves 450 mm internal dia of sieve sets as per BIS of required sieve sizes complete with lid and pan; 1 set. b); IS sieve 200 mm internal dia (brass frame and steel/or brass wire cloth mesh) consisting of sieve sets of required sieve sizes complete with lid and pan; 2 sets. 4; Sieve shaker capable of shaking 200 mm and 450 mm dia sieves-electrically operated with time switch Assembly; 1 No. 5; 200 tonnes compression testing machine; 1 No. 6; Stop watches 1/5 sec. accuracy; 2 Nos. 7; Glassware comprising beakers, pipettes, dishes, measuring cylinders (100 to 1000 cc capacity) glass rods and funnels, glass thermometers range 0°C to 100°C and metallic thermometers range up to 300°C.; 4 No. each. 8; Hot plates 200 mm dia (1500 watt.); 1 No. 9; Enamel trays. a) 600 mm x 450 mm x 50 mm; 2 Nos. b) 450 mm x 300 mm x 40 mm; 2 Nos. c) 300 mm x 250 mm x 40 mm; 2 Nos. d) Circular plates of 250 mm dia; 2 Nos. 10; Water Testing Kit; 2 No. B; FOR SOILS. 1; Water still/bath; 2 No.. 2; Liquid limit device with ASTM grooving tools as per IS:2720; 1 No. 3; Sampling pipettes fitted with pressure and suction inlets, 10 ml. Capacity; 1 set. 4; Compaction apparatus (Proctor) as per IS:2720 (Part 5) (Part 7) complete with collar, base plate and hammer and all other accessories; 1 No. 5; Modified AASHTO Compaction apparatus as per IS:2720 (Part 8) or Heavy Compaction Apparatus as per IS complete with collar, base plate hammer and all other accessories; 4 Nos. 6; Sand pouring cylinder with conical funnel and tap and complete as per IS:2720 (Part 28 ) including modern equipment.; 2 Nos. 7; Ennore/ Natural Standard Sand passing through 1 mm IS Sieve and retained on 0.600 mm/600 µ IS Sieve as per IS 2720 Part 28.; As required. 8; Sampling tins with lids 100 mm dia x 75 mm ht. ½ kg capacity and miscellaneous items like moisture tins with lid 50 grams etc.; 8 Nos each. 9; Lab CBR testing equipment for conducting CBR testing, load frame with 5 Tonne capacity, electrically operated with speed control as per IS 2720 (Part 16) and consisting of following:; 1 Set. a); CBR moulds 150 mm dia – 175 mm height. Complete with collar, base plate etc.; 6 No. b); Tripod stands for holding dial gauge holder; 4 Nos. c); CBR plunger with settlement dial gauge holder; 1 No. d); Surcharge weight 147 mm dia 2.5 kg wt. With central hole; 6 Nos. e); Spacers disc 148 mm dia 47.7 mm height. With Handle; 2 Nos. f); Perforated plate (Brass); 6 Nos. g); Soaking tank for accommodating 6 CBR moulds Proving rings of 1000 kg, 2500 kg capacity; 2 Nos. h); Proving rings 1000 kg (10 kN) , 2500 kg (25 kN) and 3000 kg (30kN) capacity as per IRC:SP:112.; 1 No each. i); Dial gauges 25 mm travel – 0.01 mm/division; 4 No. 10; Dynamic Cone Penetration test equipment; 2 Nos. 11; Standard penetration test equipment; 1 No. 12; Moisture Density meter or equivalent; 1 No.. 13; Speedy moisture meter complete with chemicals; 2 No. 14; Rifle Box; 1 No. 15; Unconfined Compression Test Apparatus; 1 No. 16; Direct shear test apparatus having 12 variables speed and 2 kN proving ring with all the accessories as given in IS 2720 Part 13.; 1 No. 17; Differential free swell index apparatus as per IS 2720 Part 40 as per IRC:SP:112; 1 No.. 18; Hydrometer and Hydrometer jar as per IRC:SP:112; 2 No.. 19; Differential Free Swell Index Apparatus as per IS 2720/ Part 40, as per IRC:SP:112; 1 No.. C; FOR BITUMEN AND BITUMINOUS MIXES. 1; Constant temperature bath for accommodating bitumen test specimen, electrically operated, and thermostatically controlled (to accommodate minimum six Specimens); 1 No. 2; Penetrometer automatic type, including adjustable weight arrangement and needles as per IS:1203; 1 No. 3; Soxhlet extraction or centrifuge type apparatus complete with extraction thimbles with solvent and filter paper; 1 No. 4; Bitumen laboratory mixer including required accessories (20 ltrs.); 1 No. 5; Marshall compaction apparatus/ Marshall Stability test apparatus automatically operated as per IS: 17127/ ASTM 1559/ ASTM – D 6927 complete with accessories (with 180 N Marshall Moulds) “ with 25 kN and 50 kN proving ring and automatic compactor and mould for Marshall and modified Marshall test”.; 1 set. 6; Furol viscometer; 1 No. 7; Ductility meter; 1 No. 8; Softening point (Ring and ball app); 1 No. 9; Distant reading thermometer; –. 10; Rifle box; – 1 No. 11; Automatic Asphalt content Meter; – 1 No. 12; Thin film oven test apparatus for modified binder either with PMB or CRMB; –. 13; Mastic Asphalt Hardness testing equipment; –. 14; Sand Equivalent test apparatus; 1 set. 15; Core cutting machine suitable for upto 150 mm dia core; 1 set. 16; Thermometers and Distant reading thermometers; 4 Nos each. 17; Cannon Manning Viscometer for determination of Absolute Viscosity and Cannon Fenske Viscometer for determination of kinematic viscosity.; 1 No.. 18; Gas stove and cylinder for testing bitumen; 1 No.. D; FOR CEMENT, CEMENT CONCRETE AND MATERIALS. 1; Water still/bath; 1 No. 2; Vicat needle apparatus for setting time with plungers as IS:269/ IS 5513; 1 No. 3; Steel Moulds IS: 10086. a)150 mm x 300 mm height Cylinder with capping component along with the capping set and compound as per IS; Minimum 6 Nos /As req. b) Cube 150 mm, and 100 mm (each size); Minimum 6 Nos /As req. 4; Concrete permeability apparatus as per DIN:1048 Part 5 MORTH 1717.7.5; 1 No. 5; High frequency mortar cube vibration machine (IS:10080) for cement testing; 1 No. 6; Concrete mixer power driven, 0.027 cum capacity IS 4925; 1 No. 7; Variable frequency and amplitude vibrating table size 1 m x 1 m as per the relevant IS/ 7246/British Standard; 1 No. 8; Flakiness and elongation index test apparatus; 1 No each. 9; Aggregate impact test apparatus as per IS:9377/IS:2386 (Part 4); 1 No. 10; Los-Angeles abrasion test apparatus as per IS:2386 (Part 4); 1 No. 11; Flow table as per IS:712; 1 No. 12; Equipment for slump test IS:7320; 4 No. 13; Equipment for determination of specific gravity or fine and coarse aggregate as perI S:2386 (Part 3) 1963; 1 No. 14; Compression and Flexural strength testing machine of 200 T capacity with additional dial gauge for flexural testing; 1 No. 15; Core cutting machine with 100 mm, 50/60mm, 75 mm and 150 mm dia diamond cutting edge with cutting edges/bits minimum two numbers each; 2 No each. 16; Needle vibrator 25 mm, 40 mm, 60 mm, 75 mm and 90 mm, IS 3558; 2 Nos each. 17; Air entrainment meter for fresh concrete IS 1199 Part 4; 1 No. 18; 3 litre, 15 litre and 30 litre cylinder for checking bulk density of aggregate with tamping rod. as per IS 2386 part III; As req. 19; Soundness testing apparatus for cement (Le-Chatelier) IS 4031/ IS : 5514; 1 set. 20; Cement motor test cube moulds 70.6 mm x 70.6 mm X 70.6 mm IS: 10086; 9 No.. 21; Aggregate Stone Polishing Test Equipment. IS 2386 Part 4, British Pendulum Tester (BPT); 1 No. E; EQUIPMENTS REQUIRED FOR FIELD AND FOR CONTROL OF PROFILE AND SURFACE EVENNESS. 1; Total Station with traversing target, having an accuracy of one second.; 1 No. 2; Precision automatic level with micrometer attachment; 1 set. 3; Distomat or equivalent; 1 set. 4; Theodolite – Electronically operated with computerised output attachment; 1 set. 5; Precision staff; 2 sets. 6; 3 meter straight edge and measuring wedge as per IRCSP:16; 2 sets each. 7; Camber template 2 Lane. a) Crown type cross-section; 1 set. b) Straight run cross-section; 2 sets. 8; Steel tape. a) 5 m long; 2 Nos. b) 10 m long; 2 Nos. c) 20 m long; 2 Nos. d) 30 m long; 2 Nos. e) 50 m long; 1 No. 9; Benkelman beam deflectometer IRC 81; 1 No ( when required). 10; Rougho-meter/ Fifth Wheel Bump Integrator (FWBI)/ Automated Road Unevenness Recorder (ARUR) as per IRCSP:16 or other equipment such as Car Axle Mounted Bump Integrator/Laser Profilometer (LP) as per the requirement as per IRCSP:16; 1 No. (when required). 11; Falling Weight Deflectomer (FWD) as per IRC:115 and IRC:117; 1 No (when required). 12; Network Survey Vehicle (NSV) as per IRC:SP:16, IRC:134 and IRC:82; 1 No (When required). 13; GPR Ground Penetration radar as per IRC 112; 1 No (When required). 14; Maturity meter test as pr IRC 112 (ASTM C 1074).; 1 No (When required). 15; Rebound Hammer test IS 456/IS:516; 1 No (When required). 16; Penetration Probe Test as per IRC 112/ (ASTM C 803),; 1 No (When required). 17; Pullout test as per IRC112/ (ASTM C 900); 1 No (When required). 18; Ultrasonic Pulse Velocity (UPV) Meter as per IS:516; 1 No (When required). F; TRAFFIC SIGN MARKING AND OTHER ROAD APPURTENANCES. 1; Retro Reflecto Meter for Sign Board as per IRC:67; 1. 2; Equipment for testing thickness and skid resistance of thermo plastic marking as per IRC:35; 1. Note:- The items and their numbers listed above in this Section are indicative and shall be decided by the Engineer as per requirements of the Project and modified accordingly. 123.5 Rate Provision and maintenance of the field laboratory is not a payable item as it is incidental to the work. - SUPPLY OF PROJECT RECORD - Scope The work covers the supply of digital record of project events in digital format including coloured photographs both in digital format as well as mounted on albums to serve as a permanent record of the work needed for an authentic documentation, as approved by the Engineer. - Description The Contractor shall provide the following project records in digital format as directed by the Engineer : - Record of work in each work front : It shall cover the status of each workfront before start of work, during various stages of construction and after completion duly including the arrangements made (day & night) for traffic during construction (This shall be need based or as directed by the Engineer); - Record of quarry sites, plant sites, camp sites including labour camps, haul roads, access roads, etc. on quarterly basis; - Record of all accidents on project road/ various sites (quarry, plant, camp, etc.) The record shall be taken by a professional with a digital camera capable of taking still as well as video images having the facility to record the date and the background commentary. The Contractor shall keep separate discs/drives, one with the Engineer and the other with the Employer and update the data in these discs/drives on monthly basis. Separately, a video (in digital format) of maximum one hour duration covering interesting and novel features of the work duly editing the above master disc/drive shall also be maintained, one copy each kept with the Engineer and the Employer and updated on monthly basis. All recording shall be done in the presence of the Engineer’s Representative who will certify in writing the recording. - Rate Supply of project record in digital format in two copies (one for the Engineer and the other for the Employer) including video recordings updated on monthly basis throughout the construction period shall be measured as one single item. Appendix 1 (Refer clause 101) LIST OF IRC PUBLICATION REVISED UPTO 2024 REFERRED TO IN SPECIFICATIONS IRC:5-2024 Standard Specifications and Code of Practice for Road Bridges, Section I – General Features of Design (Ninth Revision) IRC:6-2017 Standard Specifications and Code of Practice for Road Bridges, Section-II Loads and Load Combinations (Seventh Revision) IRC:7-2017 Recommended Practice for Numbering Culverts, Bridges and Tunnels (Second Revision) IRC:11-2015 Recommended Practice for the Design and Layout of Cycle Tracks (First Revision) IRC:12-2016 Unified Guidelines for Access Permission to Fuel Stations, Private Properties, Rest Area Complexes and Such Other Facilities Along National Highways (Fourth Revision) IRC:15-2017 Code of Practice for Construction of Jointefd Plain Concrete Pavements (Fifth Revision) IRC:22-2015 Standard Specifications and Code of Practice for Road Bridges, Section VI – Composite Construction (Limit States Design) (Third Revision) IRC:35-2015 Code of Practice for Road Markings (Second Revision) IRC:37-2018 Guidelines for the Design of Flexible Pavements (Fourth Revision) (With CD) IRC:43-2015 Recommended Practice for Plants, Tools and Equipment Required for Construction and Maintenance of Concrete Roads (First Revision) IRC:44-2017 Guidelines for Cement Concrete Mix Design for Pavements (Third Revision) IRC:52-2019 Guidelines for the Alignment Survey and Geometric Design of Hill Roads (Third Revision) IRC:65-2017 Guidelines for Planning and Design of Roundabouts (First Revision) IRC: 67-2022 Code of Practice for Road Signs (Fourth Revision) IRC:70-2017 Regulation and Control of Mixed Traffic in Urban Areas (first revision) IRC:75-2015 Guidelines for the Design of High Embankments (First Revision) IRC:78-2024 Standard Specifications and Code of Practice for Road Bridges, Foundations and Substructure Part I – General Features and Working Stress Design (Third Revision) IRC:79-2019 Recommended Practice for Road Delineators (First Revision) IRC:82-2023 Code of Practice for Maintenance of Bituminous Road (Second Revision) IRC:83-2015 Standard Specifications and Code of Practice for Road Bridges, Section IX Bearings, Part I : Roller & Rocker Bearings (Second Revision) IRC:83-2018 (Part II) Standard Specifications and Code of Practice for Road Bridges, Section IX – Bearings (Elastomeric Bearings), Part II (Second Revision) IRC:83-2018 (Part III) Standard Specifications and Code of Practice for Road Bridges, Section IX – Bearings, Part III: POT, PIN, Metallic Guide and Plane Sliding Bearings (First Revision) IRC:83-2014 (Part IV) Standard Specifications and Code of Practice for Road Bridges, Section IX – Bearings (Spherical and Cylindrical) (First Revision) IRC:85-2015 Recommended Practice for Accelerated Strength Testing & Evaluation of Concrete (First Revision) IRC:86-2018 Geometric Design Standards for Urban Roads and Streets (First Revision) IRC:87-2018 Guidelines for Formwork, Falsework and Temporary Structures for Road Bridges (Second Revision) IRC:89-2019 Guidelines for Design and Construction of River Training and Control Works for Road Bridges (Second Revision) IRC:92-2017 Guidelines for the Design of Interchanges in Urban Areas (First Revision) IRC:99-2018 Guidelines for Traffic Calming Measures in Urban and Rural Areas (First Revision) IRC:107-2013 Specifications for Bitumen Mastic Wearing Courses (First Revision) IRC:108-2015 Guidelines for Traffic Forecast on Highways (First Revision) IRC:109-2015 Guidelines for Wet Mix Macadam (First Revision) IRC:112-2020 Code of Practice for Concrete Road Bridges (First Revision) IRC:113-2013 Guidelines for the Design and Construction of Geosynthetic Reinforced Embankments on Soft Subsoils IRC:114-2013 Guidelines for use of Silica Fume in Rigid Pavements IRC:115-2014 Guidelines for Structural Evaluation and Strengthening of Flexible Road Pavements Using Falling Weight Deflectometer (FWD) Technique (with CD) IRC:116-2014 Specifications for Readymade Bituminous Pothole Paching Mix using Cut-Back Bitumen IRC:117-2015 Guidelines for the Structural Evaluation of Rigid Pavement Falling Weight Deflectometer (with CD) IRC:118-2015 Guidelines for Design and Construction of Continuously Reinforced Concrete Pavement (CRCP) IRC:119-2015 Guidelines for Traffic Safety Barriers IRC:120-2015 Recommended Practice for Recycling of Bituminous Pavements IRC:121-2017 Guidelines for Use of Construction and Demolition Waste in Road Sector IRC:122-2017 Guidelines for Construction of Precast Concrete Segmental Box Culverts IRC:123-2017 Guidelines on Geophysical Investigation for Bridges IRC:124-2017 Bus Rapid Transit (BRT) Design Guidelines for Indian Cities IRC:125-2017 Guidelines on Dozers for Highway Works IRC:126-2017 Guidelines on Wet Mix Plant IRC:127-2018 Guidelines on Skill Development of Workmen in Road Sector IRC:128-2019 Guidelines on Training of Highway Professionals IRC:129-2019 Specifications for Open-Graded Friction Course IRC:130-2020 Guidelines for Road Asset Management System IRC:131-2022 Guidelines for Identifying and Treating Black spots IRC:134-2022 Guidelines on Skid Resistance of Pavement IRC:137-2022 Guidelines on use of Fibre-Reinforced Polymer Bars in Road Projects (Part 1 : Glass Fibre reinforced Polymer Bars) IRC Under Publication Guidelines for Engineering of Natural Slopes IRC:SP:12-2015 Guidelines for Parking Facilities in Urban Roads (First Revision) IRC:SP:13-2022 Guidelines for Evaluation and Design of Small Bridges and Culverts (Second Revision) IRC:SP:16-2019 Guidelines on Measuring Road Roughness and Norms (Second Revision) IRC:SP:17-2019 Guidelines for Concrete Overlays on Concrete Pavements (First Revision) IRC:SP:19-2020 Manual For Survey, Investigation and Preparation of Road Projects (Third Revision) IRC:SP:36-2018 Guidelines for IRC Standards (First Revision) IRC:SP:40-2019 Guidelines on Repair, Strengthening & Rehabilitation of Concrete Bridges (First Revision) IRC:SP:42-2014 Guidelines on Road Drainage (First Revision) IRC:SP:46-2013 Guidelines for Design and Construction of Fibre Reinforced Concrete Pavements (First Revision) IRC:SP:49-2014 Guidelines for the Use of Dry Lean Concrete as Sub-base for Rigid Pavement (First Revision) IRC SP 48-2023 Hill Road Manual (First Revision) IRC:SP:50-2013 Guidelines on Urban Drainage (First Revision) IRC:SP:51-2015 Guidelines for Load Testing of Bridges (First Revision) IRC:SP:54-2018Project Preparation Manual for Bridges (First Revision) IRC:SP:55-2014 Guidelines on Traffic Management in Work Zones (First Revision) IRC:SP:59-2019 Guidelines for Use of Geosynthetics in Road Pavements and Associated Works (First Revision) IRC:SP:62-2014 Guidelines for Design and Construction of Cement Concrete Pavements for Low Volume Roads (First Revision) (With CD) IRC:SP:63-2018Guidelines for the Use of Interlocking Concrete Block Pavement (First Revision) IRC:SP:64-2016Guidelines for the Analysis and Design of Cast-in-Place Voided Slab Superstructure (First Revision) IRC:SP:65-2018Guidelines for Design and Construction of Segmental Bridges (First Revision) IRC:SP:66-2016Guidelines for Design of Continuous Bridges (First Revision) IRC:SP:70-2016 Guidelines for the Use of High Performance Concrete (Including Self Compacting Concrete in Bridges) (First Revision) IRC:SP:71-2018Guidelines for Design and Construction of Precast Pre-Tensioned Girders for Bridges (First Revision) IRC:SP:72-2015Guidelines for the Design of Flexible Pavements for Low Volume Rural Roads (First Revision) IRC:SP:73-2018Manual of Specifications & Standards for Two Lanning of Highways with Paved Shoulders (Second Revision) IRC:SP:76-2015Guidelines for Conventional and Thin Whitetopping (First Revision) IRC:SP:83-2018Guidelines for Maintenance, Repairs & Rehabilitation of Cement Concrete Pavements (First Revision) IRC:SP-84-2019Manual for Specifications & Standards for Four Laning of Highways (Second Revision) IRC:SP-87-2019Manual of Specification & Standards for Six Laning of Highways (Second Revision) IRC:SP:88-2019Manual on Road Safety Audit (First Revision) IRC:SP-89 (Part II) – 2018 Guidelines for the Design of Stabilized Pavements (Part II) IRC:SP:91-2019Guidelines for Road Tunnels (First Revision) IRC:SP-93-2017 Guidelines on Requirements for Environmental Clearance for Road Projects IRC:SP-97-2013 Guidelines on Compaction Equipment for Roads Works IRC:SP-98-2020 Guidelines for the use of Waste Plastic in Hot Bituminous Mixes (Dry Process) in Wearing Courses (First Revision) IRC:SP-99-2023 Manual of Specifications and Standards for Expressways (First Revision) IRC:SP-100-2014Use of Cold Mix Technology in Construction and Maintenance of Roads Using Bitumen Emulsion IRC:SP:101-2019Guidelines for Warm Mix Asphalt IRC:SP-102-2014Guidelines for Design and Construction of Reinforced Soil Walls IRC:SP-103-2014Guidelines on Tree Plantation along Rural Roads IRC:SP-104-2024Guidelines for Fabrication and Erection of Steel Bridges (First Revision) IRC:SP-105-2015 Explanatory Handbook to IRC:112-2011: Code Practice for Concrete Roads Bridges IRC:SP-106-2015Engineering Guidelines on Landslide Mitigation Measures for Indian Roads IRC:SP-107-2015 Guidelines for Gap Graded Wearing Course with Rubberised Bitumen-Rubber IRC:SP-108-2015Guidelines on Preparation and Implementation of Environment Management Plan IRC:SP-109-2015Guidelines for Design and Construction of Small Diameter Piles for Road Bridges IRC:SP:110-2017 Application of Intelligent Transport System (ITS) for Urban Roads IRC:SP:111-2017 Capacity Building of Road Agencies in charge of Implementation of Road Projects in Urban Areas IRC:SP:112-2017Manual for Quality Control in Road and Bridge Works IRC:SP:113-2018 Guidelines on Flood Disaster Mitigation for Highway Engineers IRC:SP:114-2018 Guidelines for Seismic Design of Road Bridges IRC:SP:115-2018 Guidelines for Design of Integral Bridges IRC:SP:116-2018 Guidelines for Design and Installation of Gabion Structures IRC:SP:117-2018 Manual on Universal Accessibility for Urban Roads and Streets IRC:SP:119-2018 Manual of Planting and Landscaping of Urban Roads IRC:SP:120-2018 Explanatory Handbook to IRC:22-2015 “Standard Specifications and Code of Practice for Road Bridges, Section VI- Composite Construction” IRC:SP:121-2018 Guidelines for Use of Iron, Steel and Copper Slag in Construction of Rural Roads IRC:SP:122-2019 Guidelines for Green Rating of Highways IRC:SP:124-2019 Model Contract for Maintenance of Roads (Based on Single Percentage Rate) IRC:SP:125-2019 Guidelines for Cement Grouted Bituminous Mix Surfacing for Urban Roads IRC:SP:126-2019 Guidelines for Design and Construction of Low Volume Rural Roads using Jute Geotextiles IRC:SP:127-2020 Manual for Performance Assessment of Highway Assets during operations and maintenance stage IRC:SP:128-2020 Guidelines for Green Urban Roads Manual IRC:SP:132 -2022Guidelines for Use of Industrial Wastes for Construction of Subgrade and Embankments. IRC:SP:130-2022 Design and Installation of Noise Barriers for Roads. IRC:SP:131-2022 Guidelines for Design and Evaluation of Public and Non-Motorised Transport Systems for Sustainability IRC:SP:132-2022 Guidelines on use of Industrial Wastes for Road Embankment and Subgrade Construction IRC:SP:133-2022 Guidelines on Reducing Carbon Footprint of Road Projects IRC:SP:134-2022 Guidelines for Assessment of Wave Effects on Bridges IRC:SP:135-2022 Manual for the Design of Hot Bituminous Mixes MORTH Standard Data Book for Analysis of Rates :Plain and Rolling Terrain Volume I MORTH Standard Data Book for Analysis of Rates: Hilly Terrain Volume II IRC 2018 Pocket Book for Road Construction Equipment |  |

----